SA WG2 Meeting #172	S2-2510244

17 - 21 November, 2025, Dallas, USA	(revision of S2-25xxxxx)

Source:	ZTE

Title:	KI#2 Interim Conclusion on DO-A Data Routing to AF

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The paper adds Key Issue #2 interim conclusion on AF subscription for the AIoT Device’s DO-A data.

1. Discussion

In the TR 23.700-30, several solutions propose the AF subscription for the AIoT Device’s DO-A data:

Solution #5: DO-A procedure with configured routing information:

-	AF subscribes to the DO-A Data notify by sending a Subscribe Request to the AIOTF (optionally via the NEF).

-	AIOTF derives the corresponding AF according to the routing information in the NAS DO-A Data Transfer Request message and the context generated in DO-A data subscribe procedure.

Solution #6: Support Reporting operator triggered by events without registration for DO-A capable AIoT Devices:

-	The AF subscribes to the NEF with the AF information and optional the service area. The NEF selects AIOTF(s), e.g. based on pre-configuration or the service area and subscribes to the selected AIOTF(s).

Solution #8: Service Procedure for DO-A Capable AIoT Devices:

-	This solution also proposes call flows for initiating message and DO-A data transfer message. In addition, an AF subscription procedure is provided so that the AF can subscribe the AIoT data from a specific AIOT Device.

Solution #10: DO-A Data Delivery and Routing:

-	The target address(es) of the AIoT device are stored in the AIoT Device Profile data, which are provided in the subscribe request from the AF.

Solution #11: DO-A device registration and uplink data transmission:

-	ADM is enhanced with the allowed/subscribed AF information, which is used by the AIOTF to determine the target AF for the uplink data transmission from DO-A capable device.

The paper is proposed to add the KI#2 interim conclusion regarding the routing of the AIoT Device’s DO-A data from the AIOTF to the AF based on the subscription information.

2. Proposal

It is proposed to capture the following changes into TR 23.700-30.

* * * * Start of Changes (All new text) * * * *

7.1.2	Agreed Principles for KI#2

The following principles are agreed for Key Issue #2.

DO-A data routing to AF:

-	The DO-A data of AIoT Device is routed from the AIOTF to the AF based on the subscription from the AF.

-	The AF subscribes to the AIOTF (directly for the trusted AF, through NEF for the trusted AF) for the DO-A data from the AIoT Device.

-	The AIOTF authorizes the subscription request from the AF.

-	The AIOTF interacts with the ADM to store and manage the subscription information for the DO-A data from the AIoT Device.

* * * * End of Changes * * * *