SA WG2 Meeting #172	S2-2510285

17 - 21 October, 2025, Dallas, US

Source:	InterDigital Inc

Title: 	KI #3, Key Issue #3 Conclusion

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes a conclusion for KI#3.

# 1.	Introduction

## 1.1 Proposal #1 - Schedule of EnergySaving/not EnergySaving

At SA2 #171, topics for further consideration for KI#3 were approved in clause 7.2.3 of TR 23.700-67. The following text is included for further consideration.

[Start of text from clause 7.2.3 of TR 23.700-67]

-	For NF selection, the NF profile is enhanced to include the parameters in table 7.2.3-1.

Table 7.2.3-1

| Information | Description |
| --- | --- |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

[End of text from clause 7.2.3 of TR 23.700-67]

This contribution also proposes to agree to enhance the NF profile to include the EnergySaving/notEnergySaving state to assist with NF selection, and to include this enhancement as part of KI#3 agreed principles.

An NF in EnergySaving state cannot serve requests other than those from OAM, e.g., to return to notEnergySaving state. In one example, such as when using SSC mode 1 for PDU Sessions, it is desirable that NFs, i.e., SMF, UPF, N3IWF or TNGF that undergo planned EnergySaving state, are not selected to serve such PDU Sessions.

Similarly, when an NF is known to be imminently entering EnergySaving state, it should not be selected by a NF consumer.

Furthermore, when there is a planned schedule when a NF will be in EnergySaving state, the schedule of the planned EnergySaving state can be helpful information for NF consumers so that, e.g., NF consumers can avoid selecting NFs that could not be available at certain times of the day, due to EnergySaving.

Hence, it is useful to include the EnergySaving/notEnergySaving state and related scheduling, in the NF profile.

## 1.2 Proposal #2 - N3IWF or TNGF reselection

At SA2 #171, the following text was also added to clause 7.2.3 of TR 23.700-67 for further consideration under KI#3.

[Start of text from clause 7.2.3 of TR 23.700-67]

The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. N3IWF and TNGF profiles are be provided to the NRF via OAM.

[End of text from clause 7.2.3 of TR 23.700-67]

The text above is based on Solution #29. This p-CR proposes to add this text to the agreed principles and conclusion so that an N3IWF or TNGF can be selected based on Energy Priority Information and/or Schedule of EnergySaving/not EnergySaving.

In the Rel-19 specifications, the AMF cannot consider energy information when deciding whether to redirect the UE to a different TNGF or N3IWF. Selection of the N3IWF and TNGF influences what UPFs can eventually be selected when PDU Sessions are established. This enhancement will further optimize user plane energy consumption by allowing for energy information to be configured for N3IWFs and TNGFs and considered by the AMF.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67 V1.1.0.

# * * * * First Change * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in table 7.1.3-1.

Table 7.1.3-1

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy Priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses the Energy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-	The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. N3IWF and TNGF profiles are be provided to the NRF via OAM.

# * * * * Next Change * * * *

### 7.2.3	Topics for further consideration for KI#3

The following principles are for further discussions for KI#3:

-	UP path(s) adjustment for PDU Session(s) may be triggered via DNAI (re)selection, taking into account energy related information, e.g. the overall energy consumption and/or energy efficiency of the UP path(s) for transmitting the service data flow.

-	In case of multiple configured energy policies in network, the Energy Priority is an array of priorities. Each entry of the array is the priority of the NF in the corresponding energy policy configured by the operator.

# * * * * Next Change (All New Text) * * * *

### 8.3	Conclusions for KI#3

The following principles are concluded for KI#3:

-	For NF selection, the NF profile is enhanced to include the parameters in table 8.3-1

Table 8.3-1

| Information | Description |
| --- | --- |
| Energy Priority Information | Value of Energy Priority Set by the operator (e.g. taking into account various aspects such as usage of renewable energy or other energy related information outside the scope of this document). It indicates the priority relative to other NFs of the same type. from an energy standpoint. For instance, if the Priority attribute is also used in the NF profile, the Energy Priority indicates a further priority based on energy among NFs with the same Priority attribute value in the NF profile. Whether the operator chooses the Energy Priority or the Priority attribute as primary evaluation criterion is up to the operator. |
| Schedule of EnergySaving/not EnergySaving | The schedule of the EnergySaving/not EnergySaving states (NOTE 1), includes the EnergySaving or not EnergySaving state which applies at each time interval in the schedule. |
| NOTE 1: EnergySaving/not EnergySaving refers to states of a network function which are defined in TS 28.310 [9]. |  |

-	The schedule information may be taken into account in the process of the selection of the NF by the consumer of the information in order to select the appropriate NF considering the future behaviour, if needed. The criteria for taking into account the schedule information is implementation specific.

-	For UP path selection and reselection, the UPFs are selected/reselected by using NF discovery and selection procedures which may be Energy-aware if NF profile is updated as per bullets above. Changes in NF profile of a UPF may trigger UPF reselection.

-	The NF Discovery procedure, based on operator policy, takes into account any Energy-related NF profile attributes query parameters. Both the NRF and the NF service consumer can take into account the Energy-related NF profile attributes if available, as part of the NF Discovery procedure.

-	A NF service consumer takes into account any Energy-related NF profile attributes from the NF profiles discovered from NRF using the NF Discovery procedure (and subsequent updates if the NF subscribed to NF profile updates), for the selection of a target NF.

-	The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. N3IWF and TNGF profiles are be provided to the NRF via OAM.

# * * * * End of change * * * *