SA WG2 Meeting #S2-172	S2-2510295

Dallas, USA, November 17 – November 21, 2025

Source:	Samsung

Title:	Conclusions for KI#2 UPF reporting load reduction and UC#2

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes the conclusions for UPF reporting load reduction and AIML_CN KI#2 UC#2.

# Proposal

It is proposed to adopt the following changes into TR 23.700-04.

****************Start of the change ****************

#### 7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF):

-	the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288 [5]) for input data filtering to NWDAF in analytics request. Then the NWDAF may include the necessary information in the Analytics Filter Information.

-	UPF may combine the measurement data related to multiple event ID(s) into one notify message and send a combined notification to NWDAF.

-	SMF may instruct rules to UPF for pre-processing data in UPF before reporting data to NWDAF.

****************Next change ****************

#### 7.1.2.3	Agreed Principles for Use Case #2

-	The service consumer of the analytics service is PCF. The PCF may take the analytics outputs into account to derive the QoS for the target application(s).

-	The NWDAF may provide the output analytics on traffic pattern information, including:

-	Maximum Burst Size/ Maximum Data Burst Volume, Maximum Bitrate, Guaranteed Bitrate, 5GS delay, Packet Error Rate, Periodicity.

-	Corresponding information of the traffic flow, e.g. the Application ID and/or Packet Filter Set, identifier of the service data flow(s) of the target application(s).

-	Identifiers of affected UEs, e.g. UE ID(s).

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow(s), e.g. packet filter, application ID of the traffic flow.

-	Information identifying the associated UEs.

-	Information related to the data flow characteristics, including, N6 delay, congestion information, indication that UPF drops packets and packet drop rate, data volume, packet delay, data rate, periodicity, and packet interval.

NOTE 1:	The indication of UPF dropping packets and packet drop rate is only applied to the abnormal scenarios and the scenarios when UPF dropped buffered data.

NOTE 2:	Whether the "User Data Usage Measures" event or another existing event is extended or new event(s) are defined for NWDAF data collection will be determined in the normative phase.

-	The consumer PCF may take the output Analytics into account, e.g. to generate or update PCC rules by adjusting the QoS.

-	The analytics consumer may include the Analytics Filter Information in the request, such as application description, UE ID/address, DNN and S-NSSAI, etc.

NOTE 3:	 the detailed information included in Analytics Filter Information will be determined in normative phase.

****************Next change ****************

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

## 8.x	Conclusions for KI#2

### 8.x.x General

To reduce the reporting load of input data sources (e.g. UPF):

-	the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288 [5]) for input data filtering to NWDAF in analytics request. Then the NWDAF may include the necessary information in the Analytics Filter Information.

-	the UPF may combine the measurement data related to multiple event ID(s) into one notify message and send a combined notification to NWDAF.

-	SMF may instruct rules to UPF for pre-processing data in UPF before reporting data to NWDAF.

### 8.x.y Conclusions for KI#2 Use Case#2

The following conclusions for KI#2 Use Case#2 will be supported by normative work:

-	The service consumer of the analytics service is PCF. The consumer PCF may take the output Analytics into account, e.g. to generate or update PCC rules by adjusting the QoS.

-	The NWDAF may provide the output analytics on traffic pattern information to the consumer, including:

-	Maximum Burst Size/ Maximum Data Burst Volume, Maximum Bitrate, Guaranteed Bitrate, 5GS delay, Packet Error Rate, Periodicity.

-	Corresponding information of the traffic flow, e.g. the Application ID and/or Packet Filter Set, identifier of the service data flow(s) of the target application(s).

-	Identifiers of affected UEs, e.g. UE ID(s).

-	To derive the output analytics, the following input data may be collected from NFs to support the NWDAF-based analytics:

-	Information identifying the associated application/service data flow(s), e.g. packet filter, application ID of the traffic flow.

-	Information identifying the associated UEs.

-	Information related to the data flow characteristics, including, N6 delay, congestion information, indication that UPF drops packets and packet drop rate, data volume, packet delay, data rate, periodicity, and packet interval.

NOTE 1:	The indication of UPF dropping packets and packet drop rate is only applied to the abnormal scenarios and the scenarios when UPF dropped buffered data.

NOTE 2:	Whether the "User Data Usage Measures" event or another existing event is extended or new event(s) are defined for NWDAF data collection will be determined in the normative phase.

-	The analytics consumer may include the Analytics Filter Information in the request, such as application description, UE ID/address, DNN and S-NSSAI, etc.

NOTE 3:	 the detailed information included in Analytics Filter Information will be determined in normative phase.

****************End of the change ****************