3GPP TSG-SA WG2 Meeting #172	S2-2510378

17- 21 November, 2025, Dallas, US(revision

Source: 	ZTE, MediaTek Inc., CMCC, Huawei, Nokia

Title: 	Interim conclusion for KI#1 on IMS voice packet transport mechanism

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes an interim conclusion for KI#1 of FS_5GSAT_Ph4_ARC regarding the evaluation evaluation of IP-based and non-IP-based voice packet transmission approaches.

1	Discussion

In TR 23.700-19, a total of thirteen candidate solutions are identified to support IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, covering both IP-based and non-IP-based voice packet transmission mechanisms.

The discussion paper S2-2510377 and S2-2507102 analyze the pros and cons of IP‑based and non‑IP‑based voice packet transport mechanisms and lead to the following conclusions:

Compared to Non-IP (Reduced RTP), The IP (RoHC) approach:

- can achieve comparable or lower overhead

- is significantly more future-proof (can natively support DTMF, MULTI-CALL/FEC, JBM, etc.)

- has fewer dependencies on SA1, and no additional work for SA3 (Security work), and SA4 (Reduced RTP work)

- requires much fewer enhancements to existing system

- leverages the widely commercial deployed VoLTE design to the maximum.

- provides higher reliability, stronger security, and better interoperability

- RoHC issues are either un-likely to happen, or implementation‑specific and can be resolved through proper implementation.

It is important to note that the architectural assumptions for IMS voice over NB‑IoT (GEO) explicitly state that enhancements to the NB‑IoT (GEO), EPC, and IMS components shall be minimized in supporting IMS voice service via GEO satellite.

In this context, our analysis shows that the non‑IP solution would require extensive network modifications and cross‑WG new-design efforts, while achieving performance only comparable to—or even worse than—the IP‑based approach.

Given these findings, it is therefore proposed to reach interim consensus on adopting the IP‑based voice transport option as the preferred direction.

2 Proposal

It is proposed to agree and update TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows.

# * * * * First Change* * *

8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	RTP/UDP/IP with Robust Header Compression (RoHC) shall be supported for transporting voice packets.

# * * * * End of change * * * *