SA WG2 Meeting #S2-172	S2-2510387

17 - 21 November, 2025, Dallas, USA

Source: 	vivo

Title: 	Conclusions on KI#1: System Architecture to Support Sensing

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides Conclusion on KI#1: System Architecture to Support Sensing.

1	Discussion

At June 2025 SA#108 meeting, R20 5GA SA2 ISAC (see approved SID SP-250833) was down-scoped into gNB-based sensing to aligned with Rel-20 5G-A RAN ISAC study scope. Based on this, we could have:

Observation#1: Only gNB can be performed as Sensing Entity (SE).

Up to August 2025 SA2#171 meeting, 12 Solutions was agreed for KI#1 and captured in TR 23.700-14 V1.1.0.

After analysing and evaluating the architecture proposals, new NF function definitions, existing NF impacts and end-to-end procedure definitions captured in agreed solutions for KI#1, following common aspects can be identified and could be captured as agreed principle for KI#1:

Observation#2: A new CN function Sensing Function (SF) is introduced to support Sensing service which provides following key functionalities:

- NF profile registration for the discovery of SF instances, e.g., by an NEF.

- Handling Sensing service request from Sensing service consumer.

- Sensing service request authorization and revocation.

- Sensing Entity selection and re-selection.

- Sensing data processing and Sensing result generation/exposure.

- Sensing configuration parameters provision.

Observation#3: The Sensing Entity (i.e. gNB) provides following key functionalities:

- Sensing service operations to collect the Sensing data.

- Sensing data report to the SF.

Observation#4: The NEF, if involved, needs to support following key functionalities:

- Handling Sensing service request from the AF.

- SF discovery and selection.

- Sensing service request authorization.

Observation#5: Both direct connection architecture (SE-SF) and indirect connection architecture (SE-AMF-SF) are considered for 5G system architecture to support sensing.

It proposes to capture above observations as interim agreements on KI#1: System Architecture to Support Sensing.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-14.

# * * * * First change* * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

8.X Conclusions for KI#1: System Architecture to Support Sensing

To resolve the KI#1: System Architecture to Support Sensing, the following conclusions are adopted for normative work:

a)	one new Network Function (i.e. Sensing Function, SF) is defined to support Sensing Service. the SF may contain Sensing control functionality (SCF) and Sensing Processing functionality (SPF).

NOTE 1:	There is no standardized interface between the SCF and SPF in Rel-20 5G-A.

NOTE 2:	How to capture the deployment options of SPF(s) functionality and SCF functionality of the same SF, e.g. co-located or separated, can be discussed during the normative phase.

NOTE 3:	The final name and Acronym for Sensing Function may need further update if necessary.

b)	NO dedicated storage NF is needed to store the Sensing data and Sensing result.

c)	In this study, the gNB is the only entity that acts as the Sensing Entity (SE).

d)	There are two interfaces between the SF and the SE, one for Sensing control messages exchange and one for Sensing Data transport.

e)	Sensing control messages are exchanged between the SF and the SE either directly or via the AMF indirectly.

f)	Sensing data is transported from the SE to the SF directly.

g)	Except the NEF, NO dedicated NF is needed for SF discovery and selection for Sensing service request.

NOTE 4:	Privacy protection and other security aspects will be coordinated with SA WG3 and the related impact to architecture enhancement will be based on SA WG3 during the normative phase.

NOTE 5:	Other KI's interim conclusions will be aligned with KI#1 conclusions.

# * * * * End of change* * * *