3GPP TSG-SA WG2 Meeting #172	S2-2510446

17 – 21, November, 2025, Dallas, USA	(revision of S2-25xxxxx)

Source:	Xiaomi

Title:	Solution #2 update to remove Editor’s note

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes to update solution#2 to remove Editor’s note.

1. Introduction/Discussion

In solution#2, an Editor’s note is left to resolve:

Editor's note:	Whether and how to resolve the conflict with maximum 2-DRB restriction if concurrently supporting data and IMS services for NB-IoT UEs is FFS and needs coordination with RAN WGs.

According to the LS reply from RAN2 (R2-2507784/S2-2509932), this EN can be resolved.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

RAN2 reply to Question 6: RAN2 understands that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

This contribution proposes to update solution#2 to remove the Editor’s note.

2. Proposal

It is proposed to capture the following update in FS_5GSAT_Ph4_ARC TR 23.700-19 v1.1.0.

* * * Start of Change * * * *

6.2.2.2	 MO call

The following figure depicts the mobile originating procedure for IMS session establishment over the UP-based NB-IoT (GEO) deployment.

Figure 6.2.2.2-1: MO call Procedure

1.	The MO UE camps on the eNB that supports NB-IoT (GEO) satellite access and performs attach procedure and establishes the PDN connection for IMS service. The detailed procedure can refer to clause 6.2.3.1. Optionally, the dedicated EPS bearer for IMS voice can be established.

2.	The MO UE performs IMS registration procedure as specified in clause 5.2.2 of TS 23.228 [6], which is via the default EPS bearer of the IMS PDN connection. P-CSCF can obtain RAT type of the IP-CAN by subscribing to the IP-CAN Type Change with PCRF during the SIP registration, as described in Annex B, clause B.1a of TS 29.213 [11].

3-4.	The MO UE initiates a voice call using SIP INVITE message with an SDP offer towards the P-CSCF, which is over the default EPS bearer of the IMS PDN connection. The P-CSCF forwards the SIP INVITE message towards IMS core and receives SIP 183 response with an SDP answer to the SIP INVITE message as specified in clause 5.6 of TS 23.228 [6].

NOTE:	Any IMS optimizations, including compression or substitution mechanisms, will be addressed in Key Issue#2 study.

5-6.	The P-CSCF sends diameter AAR according to the SDP offer and the SDP answer received to the PCRF as specified in clause 8.1 of TS 29.211 [10]. The PCRF sends a PCC decision provision (QoS policy) message to the PDN GW as specified in step 1 of clause 5.4.1 in TS 23.401 [5].

7.	If a dedicated EPS bearer is pre-established in step 1, the QoS requirement for voice data is configured for the dedicated EPS bearer. The packet filter will be updated based on the alternatives listed in clause 6.2.3.1 step 5.

If the dedicated EPS bearer is not established in step 1, the PDN GW triggers the dedicated EPS bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5], where 3 RRC messages (reconfiguration/complete/direct transfer) are used between UE and eNB.

Editor's note:	Which QoS parameters are applicable for the dedicated EPS bearer is FFS and need coordination with RAN WGs.

8-9.	The PDN GW responds to the PCRF as specified in step 12 in clause 5.4.1 of TS 23.401 [5]. The PCRF send diameter AAA to the P-CSCF as specified in clause 8.1 of TS 29.211 [10].

10.	The P-CSCF sends the SIP 183 response with the SDP answer to the MO UE, which is over the default EPS bearer of the PDN connection.

For Alt#1, the UE updates the packet filter locally based on the SDP negotiation result.

11.	The procedure is continued and the IMS session is established.

The IMS voice data are transmitted over the dedicated EPS bearer.

* * * End of Change * * * *