3GPP TSG-WG SA2 Meeting #172 	S2-2510571

Dallas, U.S.A, Nov 17th – 21st, 2025

Source:	NTT DOCOMO

Title:	Update: Interim agreements for FS_Sensing_ARC KI#3

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: This contribution proposes to update the Interim agreements for KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection.

# 1. Discussion

In TR 23.700‑14 v1.1.0, the current agreed principles for Key Issue #3 (Sensing Entity and Sensing Function Discovery and (Re-)Selection) provide a foundation for the discovery and selection of Sensing Functions and Sensing Entities. However, the aspect of re-selection, although explicitly mentioned in the description of KI#3 in clause 5.3 of the TR 23.700‑14 v1.1.0, is not yet reflected in the agreed principles.

Clause 5.3 specifies that the study for this key issue shall address both discovery and (re-)selection of Sensing Functions and Sensing Entities, including aspects such as the responsible entity, the criteria and information used, and the corresponding procedures. To maintain alignment with the scope and intent of the study as captured in the TR, it is therefore necessary to explicitly incorporate the re-selection aspect in the agreed principles.

The re-selection capability enables the Sensing Function to dynamically adapt to changes in the operational environment, sensing coverage, or resource availability. Such re-selection may be required when the initially selected Sensing Entities cannot fully satisfy the sensing service request, e.g. due to degradation in sensing quality or availability constraints. Allowing the Sensing Function to continue re-selecting Sensing Entities from the available set until the sensing Quality of Service (QoS) requirements are fulfilled ensures that the system can maintain the required sensing performance.

# 2. Proposal

It is proposed to update the Agreed Principles for KI#3 in clause 7.1.3 of TR 23.700‑14 v1.1.0.

# * * * * First change* * * *

### 7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing area.

-	Sensing Function (re-)selects one or more Sensing Entities, i.e. gNBs, from an available set of sensing entities, based on the sensing service request.

-	Sensing Function (re-)selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	The Sensing Function may continue to (re-)select Sensing Entities until the sensing Quality of Service (QoS) based on the sensing service request is fulfilled.

NOTE:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

# * * End of changes * * * *