3GPP TSG-SA WG2 Meeting #172		TDoc S2-2510623

Dallas, USA, 17 - 21 November, 2025

Title:	[DRAFT] LS on RTP header issue to support IMS voice over NB-IoT NTN connected to EPC

Response to:

Release:	Rel-20

Work Item:	FS_5GSAT_Ph4_ARC

Source:	NTT DOCOMO [to be: SA2]

To:	SA4

Cc:

Contact person:	Atsushi Minokuchi

minokuchi@nttdocomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

SA2 has planned to complete the study FS_5GSAT_Ph4_ARC by SA2#172 in November 2025. Given the short timeframe, SA2 has continued to study the so-called "reduced RTP header" (i.e., the short header containing sufficient information to restore the usual RTP header. The reduced RTP header is used through the path over GEO instead of the usual RTP header), while not waiting for a response from SA4 to the LS S2-2507636 that SA2 previously sent. SA2 has captured a candidate solution in TR 23.700-19, version 1.2.0, clause 6.29.

SA2 kindly seeks confirmation from SA4 that this candidate solution is feasible. Attention may need to be paid to the following points.

- The method to derive the timestamp of the RTP header based on the reduced RTP header is shown in clause 6.29.1.3.

- Values to set for P(padding) and M(marker) depends on the definition of payload format for a new codec.

- 6 bits, if HOLD/CW/DTMF is required, or otherwise 8 bits are used for sequence number in the reduced RTP header.

- RTCP is not used.

- When using HOLD/CW, RTP packets of the first call and of the second call are differentiated by the value of SSRC. When using DTMF, RTP packets for speech and for DTMF are differentiated by the value of payload type.

2	Actions

To SA4

ACTION:	SA2 kindly asks SA4 to provide any feedback on the above-mentioned candidate solution.

3	Dates of next TSG SA WG2 meetings

SA2 WG Meeting #173		February 9 - 13, 2026		Goa, India

SA2 WG Meeting #174		April 13 - 17, 2026		TBD, Malta