3GPP TSG-SA WG2 Meeting #172	S2-2510679

# November 17 - 21, 2025 Dallas, USA

Source:	Samsung

Title:	Interim agreement update and final conclusion for KI#3

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreement update and final conclusions for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection

# 1. Discussion

TS 23.700-14 v1.1.0 has some agreed principle for KI#3 Sensing Entity and Sensing Function Discovery and (Re-) but there are some other aspects that need to be addressed.

Sensing Entity discovery:

SF registers with its NF profile which contains some information like supported sensing area and used for discovery, similarly the capability of sensing entity like supported sensing area need to be available at SF for discovery of sensing entity and this capability information can be retrieved by SF via OAM.

Proposal: It is proposed that SF obtains the NG-RAN configuration information (NG-RAN node ID, supported sensing area) via OAM.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * First Change * * * *

# 7	Interim Agreements

## 7.1	Agreed Principles

### 7.1.3	Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing area.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	SF obtains the gNB configuration information (gNB node ID, supported sensing area) via OAM.

NOTE 1:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

NOTE 2:	Additional capabilities for discovery of SF and SE will be discussed during normative phase.

* * * Second Change * * * *

# 8	Conclusions

The following conclusions are agreed for KI#3:

-	The Sensing Function selection is based on NEF's local configuration or NRF query.

-	The Sensing Function registers itself to the NRF, with its NF profile.

-	Sensing Function is selected based on at least its supported sensing area.

-	Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request.

-	Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area.

-	SF obtains the gNB configuration information (gNB node ID, supported sensing area) via OAM.

NOTE 1:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

NOTE 2:	Additional capabilities for discovery of SF and SE will be discussed during normative phase.

* * * End of Changes * * * *