SA WG2 Meeting #172	S2-2510878

17 - 21 November 2025, Dallas, USA	(was S2-2510111)

Source: 	Nokia

Title: 	Interim conclusions on KI#2 Support of SMS Delivery to Emergency Response Centre via NAS

Document for: 	Approval

Agenda Item: 	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract: This contribution provides an interim conclusion of KI#2 Support of SMS Delivery to Emergency Response Centre via NAS.

# 1	Discussion

At the SA2#171 meeting, 8 solutions were proposed for Key Issue #2, forming a comprehensive set of “SMS over NAS” routing options for SMS to EC.

Given the common framework proposed in all the solution papers, this paper proposes an interim conclusion for KI#2

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-65 v0.2.0.

# * * * * First change* * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Interim conclusions for Key Issue #2

Editor's note: Other interim conclusions for Key Issue#2 are FFS.

The following interim conclusions for Key Issue #2 Support of SMS Delivery to Emergency Response Centre via NAS are made:

-	A UE receiving an indication that SMS to EC over NAS is supported in a serving PLMN but failed to send the SMS over NAS, may attempt emergency SMS over IP if configured in the UE and supported by the network.

-	If the UE need to send SMS to EC, the UE generates the SMS-SUBMIT TPDU with the local emergency number in the TP-DA field.

-	The SMSF/MME/MSC shall check the TP-DA field for the local emergency number. If the TP-DA contains a local emergency number, the SMSF/MME/MSC will replace the RP-DA with a local SMSC number earmarked to handle EM SMS as per local configuration. The SMSF/MME/MSC will forward the SMS to the GMSC/SMS-IWMSC/SMS Router after the RP-DA replacement.

-	The RP-DA replacement will also take into account the UE location provided by AMF/MME for the suitable local SMSC.

-	The local SMSC then routes the SMS to a local PSAP based on the emergency number.

# * * * * End change* * * *