3GPP TSG SA 3GPP TSG SA WG2 Meeting #79E	TD S2-103195

July 6-13, 2010, Elbonia

Title:	Follow on to LS response on support for Priority for terminating sessions for MPS

Source:	SA2

To:	RAN2, GERAN2

Cc:

Contact Person:

#### Name: 	Vijay Varma

E-mail Address:	vvarma@telcordia.com

Attachments:	None

1. Overall Description:

SA2 would like to thank RAN2 for the LS R2-104202/S2-3196 LS response on support for Priority for terminating sessions for MPS.

SA2 has noted as per RAN2 feedback that from LTE access point of view, there is no benefit from sending the priority indication in the RRC Paging message or other RRC messages over the radio for an incoming MPS session establishment attempt using IMS or CSFB. , and has agreed that during congestion conditions, network based solutions of prioritising the MPS Paging requests are sufficient.

SA2 would like to thank RAN2 for the question on the case of a mobile terminated call setup via CSFB with subsequent access over UMTS with re-direction option as per TS 23.272 clause 7.4, and is happy to provide the following response.

RAN2 Question:

RAN2 kindly asks SA2 to provide feedback on whether if it is also necessary to prioritise the subsequent access over UMTS for CSFB when Release with re-direction option as per TS 23.272 clause 7.4 is used.

SA2 Response:

SA2 concluded that it is necessary to prioritise the subsequent access over UMTS/GERAN for CSFB when Release with re-direction option is used and would like to kindly ask the RAN2/GERAN2 to consider how to ensure it.

SA2 would also like to know the implications of the above requirements on the overall priority handling, i.e. if this issue would have any impact on the decision for LTE access.

2. Actions:

To: RAN2, GERAN2

ACTION:

SA2 kindly asks RAN2 to consider how to prioritise the subsequent access over UMTS for CSFB when Release with re-direction option is used and provide feedback on the overall aspects as indicated.

SA2 kindly asks GERAN2 to provide feedback on how to prioritise the subsequent access over GERAN for CSFB when Release with re-direction option is used.

3. Date of Next TSG-SA2 Meetings

| 3GPPSA2#80 | OR | 30 Aug - 3 Sep 2010 | Brunstad | NOR |
| --- | --- | --- | --- | --- |
| 3GPPSA2#81 | OR | 11 - 15 Oct 2010 | Prague | CZE |