SA WG2 Meeting #99                                       S2-133336

23 - 27 September 2013 – Xiamen, P.R. China

Source:	NTT DOCOMO, Intel

Title:	Evaluation of C-plane congestion reporting solution

Document for:	Approval

Agenda Item:	6.6

Work Item / Release:	UPCON / Rel-12

Abstract of the contribution: This paper evaluates solution 1.5.2 for congestion reporting solutions in TR 23.705.

### 1. Introduction

This paper adds the evaluations of solution 1.5.2 for congestion reporting solution in TR 23.705.

***************** Start of 1st changes **********************

#### 6.1.5.2	Solution 1.5.2: C-plane Signalling for RAN user plane congestion reporting

##### 6.1.5.2.5	Solution evaluation

Impact on CN architecture:

- No new functional entities or interfaces needed

- Require changes to CN entities (MME, SGW, PGW, PCRF) for support of congestion reporting over C-plane

- Require small changes to CN entities (MME, SGW, PGW, PCRF) for support of policy for congestion reporting over C-plane (only in case of dynamic configuration of policy for RCI signalling)

Impact on RAN entities and interfaces:

- No new functional entities and interfaces needed

- Require changes to eNodeB and S1-MME interface for support of congestion reporting to MME

- Require small changes to eNodeB and S1-MME interface for support of policy for congestion reporting (only in case of dynamic configuration of policy for RCI signalling)

Complexity and scalability:

- No break of logic between control plane and user plane at eNodeB, MME, SGW, PGW

- No switch between U-plane and C-plane for congestion reporting from RAN to CN

- No overhead for GTP-U packets

- No processing overhead at network nodes (eNodeB, MME, SGW, PGW, PCRF) for U-plane

- Introduce additional C-plane signalling when sending a congestion reporting from the eNodeB to PCRF/PCEF

- Introduce additional C-plane signalling only when updating policy for RCI signalling of existing EPS bearer (only in case of dynamic configuration of policy for RCI signalling)

- Introduce processing overhead at network nodes (eNodeB, MME, SGW, PGW, PCRF) for C-plane, for example:

  - MME has to parse the received RCI signalling message into several messages according to the number of UE and UE’s PDN connections

Responsiveness:

- No dependency on uplink packet

- Congestion information (RCI) is up-to-date

Additional considerations (e.g., features supported or not supported):

- No proprietary interfaces or proprietary functional entities needed

- Support congestion reporting restriction for RAN sharing

- Support reporting congestion level for multiple cells under the same eNB

- Support reporting congestion level for UE simultaneously belongs to multiple cells

- Support congestion reporting restriction for home routed roaming case

- Support congestion reporting for PMIP S5/S8 case

- Support pre-provisioning policy for congestion mitigation

- Extension to information beyond congestion level is comparably easy, only requires a new AVP

***************** End of 1st changes **********************