3GPP TSG-SA3 Meeting #100e 	S3-201609

# e-meeting, 17 - 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.536 | CR | 0002 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on security establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-08-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In section 5.3.3.1.4.3, step 4, with non-NUL integrity algorithm selected the UE_1 check that the received LSBs of KNPR-sess ID is unique shall be done in the context of current DCR only. Otherwise a check for uniqueness locally, may lead UE1 to reject the DSMC message due to a LSBs of KNPR-sess ID collison with LSBs of KNPR-sess ID used by any existing link An attacker may be able to obtain knowledge of the LSBs of KNPR-sess ID and associated link identifiers in use by other UEs already connected to the target UE. - The entire KNRP-sess ID shall be used to pinpoint locally the security context since the same MSBs of KNPR-sess ID from UE_1 may be used by multiple peer UEs making the MSBs of KNPR-sess ID used alone not sufficient to uniquely identify the security context locally. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is proposed to clarify the check of LSBs of KNPR-sess ID uniqueness received in DSMC relative to current DCR - Clarify that the KNPR-sess ID is used to uniquely identify the security context locally. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The LSBs of KNPR-sess ID in use may be leaked leading to linkability/trackability attacks if the check of LSBs of KNPR-sess ID uniqueness is not done in relative to current DCR - UE is unable to establish and use a unique security context per peer UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.3.3.1.4.3	Security establishment during connection set-up

The clause describes how security is established during connection set-up. The signalling flow is shown in figure 5.3.3.1.4.3-1.

Figure 5.3.3.1.4.3-1: Security establishment at connection set-up

1.	UE_1 has sent a Direct Communication Request to UE_2. This message shall include UE_1's security capabilities (the list of algorithms that UE_1 will accept for this connection) and UE_1's signalling security policy. The UE_1 shall also include Nonce_1 (for session key KNRP-sess generation), and the most significant 8-bits of the KNRP-sess ID in this message if UE_1's signalling integrity protection policy is either "REQUIRED" or "PREFERRED". The most significant 8-bits of the KNRP-sess ID shall be chosen such that UE_1 will be able to locally identify a security context that is created by this procedure using the KNRP-sess ID. The message may also include a KNRP ID if the UE_1 has an existing KNRP for the UE that it is trying to communicate with. The absence of the KNRP ID parameter indicates that UE_1 does not have a KNRP for UE_2. The message also contains Key_Est_Info (see clause 5.3.3.1.3.2).

2.	UE_2 shall reject the Direct Communication Request if UE_1's signalling security policy is "NOT NEEDED" while UE_2's security policy is "REQUIRED". UE_2 shall also reject the Direct Communication Request if UE_1's signalling security policy is "REQUIRED" while UE_2's security policy is "NOT NEEDED". UE_2 may initiate a Direct Auth and Key Establish procedure with UE_1. This is mandatory if the UE_2 does not have the KNRP and KNRP ID pair indicated in step 1, and signalling is needed to establish the keys for the particular use case.

3.	UE_2 shall send the Direct Security Mode Command message to UE_1. This message shall only contain the MSB of KNRP ID and optionally Key_Est_Info if a fresh KNRP is to be generated (see clause 5.3.3.1.3). UE_2 shall include the Chosen_algs parameter to indicate which security algorithms the UEs will use to protect the data in the message. The Chosen-algs may only indicate the use of the NULL integrity algorithm if UE_2's signalling integrity security policy is either NOT NEEDED or PREFERRED. UE_2 shall also return the UE_1's security capabilities and UE_1's signalling security policy to provide protection against bidding down attacks. In the case that the NULL integrity algorithm is chosen, the NULL confidentiality algorithm shall also be chosen and UE_2 shall set the KNPR-sess ID of this security context to the all zero value.

The following procedures in step 3 shall only be executed if the UE_2 decides to at least activate the integrity security protection for this connection: UE_2 shall also include Nonce_2 to allow a session key to be calculated, as well as the least significant 8-bits of KNRP-sess ID in the messages. These bits are chosen so that UE_2 will be able to locally identify a security context that is created by this procedure. UE_2 shall calculate KNRP-Sess from KNRP and both Nonce_1 and Nonce_2 (see clause A.3) and then derive the confidentiality (if applicable) and integrity keys based on the chosen algorithms (clause A.2). UE_2 shall integrity protect the Direct Security Mode Command before sending it to UE_1. UE_2 is then ready to receive both signalling and user plane traffic protected with the new security context. UE_2 shall form the KNRP-sess ID from the most significant bits it received in step1 and least significant bits it sent in step3.

4.	On receiving the Direct Security Mode Command, the UE_1 shall first check the Chosen_algs and shall accept the NULL integrity algorithm only if its security policy for signalling integrity protection is either NOT NEEDED or PREFERRED. Then UE_1 shall check the returned UE_1's security capabilities and UE_1's signalling security to avoid bidding down attacks if NULL integrity algorithm is selected for signalling integrity protection. If the above check passes, UE_1 shall send unprotected Direct Security Mode Complete message to UE_2. UE_1 shall set the KNPR-sess ID of this security context to the all zero value.

Under the condition of non-NULL integrity algorithm indicated in the Chosen_algs, UE_1 shall first check that the received LSB of KNPR-sess ID is unique, by checking  that it has not been sent by another UE responding to this Direct Communication Request i.e. such that resulting KNRP-sess ID is not already being used for another link. If the LSB of KNPR-sess ID is not unique, then UE_1 shall respond with a Direct Security Mode Reject message including a cause value to specify that the LSB of KNPR-sess ID is not unique. The peer UE-2 receiving a Direct Security Mode Reject message shall inspect the cause value and, if the cause is related to the session identifier uniqueness then, the UE-2 shall generate a new LSB of KNPR-sess ID and reply to UE-1 again (i.e., UE-2 shall send a Direct Security Mode Command message with the new LSB of KNPR-sess ID). UE_2 shall associate the new LSB of KNRP-sess ID with the security context that is created in step 3. UE-2 shall erase the former LSB of KNPR-sess ID from its memory. On receiving this new Direct Security Mode Command, UE_1 shall process the message from the start of step 4.

If the LSB of KNPR-sess ID is unique, UE_1 shall calculate KNRP-sess and the confidentiality and integrity keys in the same way as UE_2. UE_1 shall check that the returned UE_1 security capabilities and UE_1's signalling security policy are the same as those it sent in step 1. UE_1 shall also check the integrity protection on the message. If both these checks pass, then UE_1 creates a security context to be associated with the KNRP-sess ID. UE_1 is ready to send and receive signalling and user plane traffic with the new security context. UE_1 shall send integrity protected and confidentiality protected Direct Security Mode Complete message to UE_2. UE_1 shall form the KNRP-sess ID from the most significant bits it sent in step1 and least significant bits it received in step3. KNRP-sess ID is used to locally identify the security context that is created by this procedure.

5.	If the Chosen_algs in step 3 includes non-NULL integrity algorithm, UE_2 checks the integrity protection on the received Direct Security Mode Complete. If this passes, UE_2 is now ready to send user plane data and control signalling protected with the new security context. UE_2 deletes any old security context it has for UE_1.

* * * End Change * * * *