3GPP TSG-SA3 Meeting #100e	S3-201692

# e-meeting, 17 -28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.536 | CR | 0006 | rev | - | Current version: | 16.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the clause 5.3.3.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the clause 5.3.3.2.2.1 and 5.3.3.2.2.2, link identifier update and Layer-2 link release procedure step 1, the MSB of KNRP-sess ID that UE_1 choose to identify KNRP-sess should not be same with the former one, as well as the LSB of KNRP-sess ID in step 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification of MSB and LSB in clause 5.3.3.2.2.1 and 5.3.3.2.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May cause some misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2.2.1, 5.3.3.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### **** START OF CHANGE 1 ****

5.3.3.2.2	Procedures

5.3.3.2.2.1	Link identifier update

Figure 5.3.3.2.2-1 shows the flows for changing the identities of the UEs involved in PC5 unicast link. The figure only displays the security parameters (KNRP-sess ID)that are changed and the Layer-2 IDs but not the other parameters described in TS 23.287 [2].

Figure 5.3.3.2.2.1-1: Link identifier update procedure

The procedure proceeds with the following steps and provides additional handling on top of what is provided in TS 23.287 [2].

0. UE_1 and UE_2 are communicating via a unicast link and have established the security for the link.

1. UE_1 decides to change its identifiers and sends a Link Identifier Update Request message to UE_2 (see TS 23.287 [2]). In addition to the changed identifiers, UE_1 shall include the MSB of KNRP-sess ID in the Link Identifier Update Request message. These bits shall be chosen so that they uniquely identify KNRP-sess at UE_1.

2. UE_2 shall choose the LSB of KNRP-sess ID so that they uniquely identify KNRP-sess at UE_2. . UE_2 shall form the new KNRP-sess ID from the MSB received from UE_1 and the LSB that UE_2 chose. UE_2 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. In addition to its updated identifiers, UE_2 shall send the LSB of KNRP-sess ID to UE_1 along with the received MSB of KNRP-sess ID and other identifiers received from UE_1 in the Link Identifier Update Response message. UE_1 shall check that the returned MSB of KNRP-sess ID is identical to the one sent in step 1. UE_1 shall check that the  received LSB of KNPR-sess ID is unique as specificed in clause 5.3.3.1.4.3 step 4.

3. UE_1 shall form the new KNRP-sess ID from the LSB received from UE_2 and the MSB chosen by UE_1 (in step 1). UE_1 shall associate the new KNRP-sess ID with the updated Layer-2 IDs (see TS 23.287 [2]) and shall use this new KNRP-sess ID when it uses the updated Layer-2 IDs. UE_1 shall send the Link Identifier Update Ack message to UE_2 including the LSB of KNRP-sess ID and other identifiers received from UE_2. UE_2 shall check that the returned LSB of KNRP-sess ID are identical to the one sent in step 2.

5.3.3.2.2.2	Layer-2 link release

Figure 5.3.3.2.2.2-2 shows the message flows for changing the KNRP ID of the UEs involved in PC5 unicast link to remediate the privacy threat for the KNRP ID. This message flow is based on the Layer-2 link release procedure provided in clause 6.3.3.3 of TS 23.287 [2]. The messages in the Layer-2 link release procedure are always sent protected and hence the new KNRP ID agreed by the UEs is only known to the involved UEs. The new KNRP ID is used on a subsequent unicast link establishment procedure (see clause 5.3.3.1.4.3).

Figure 5.3.3.2.2.2-2: Layer-2 link release procedure

0.	UE_1 and UE_2 have a unicast link established as described in TS 23.287 [2].

1.	UE_1 sends a Disconnect Request message to UE_2 in order to release the layer-2 link (see TS 23.287 [2]). UE_1 shall include the MSB of KNRP ID in the Disconnect Request message. These bits shall be chosen so that they uniquely identify KNRP at UE_1.

2.	UE_2 shall choose the LSB of KNRP ID so that they uniquely identify KNRP at UE_2.UE_2 shall form the new KNRP ID from the MSB received from UE_1 and the LSB that UE_2 chose. UE_2 may use this new KNRP ID when it reconnects with UE_1. UE_2 shall send the LSB of KNRP ID to UE_1 in the Disconnect Response message. Upon reception of the Disconnect Response message, UE_1 shall form the new KNRP ID from the LSB received from UE_2 and the MSB that was chosen by UE_1 (in step 1). UE_1 may use this new KNRP ID when it reconnects with UE_2.

**** END OF CHANGES ****