3GPP TSG-SA3 Meeting #100e 	S3-201798-r1

# e-meeting, 17 - 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 0903 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication and static authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 7.8.2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarifications needed related to which model is in use, consistant usage of accronyms CAA, NFp, NFc to avoid misspellings and allow better reading. Some wrong or not clarified NFp/NFc needs to be corrected Clause 13.3.1.3 NOTE 1 is not fitting. General clause missing. In 13.3.8.1 wrong name: NF Producer -> NFc In 13.3.8.3 missing clarification that NF Instance ID in the public key certificate as specified in 3GPP TS 33.310 needs to be present for the verification of NF instance ID in CCA. Revision 1: reverting the short names NFp/NFc to full names, reverting 13.3 to full title |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications added related to which model is in use. Consistant usage of NF Service Producer / NF Service Consumer and CAA acronym, intro in abreviation clause. In 13.3.1.2 NF -> NFc; further NOTE 1 clarification/rewrite; also NOTE 3 In 13.3.1.3 Provider -> Producer corrected; -> NFp used NOTE 1 deleted,cause not fitting here. Reference to model A and B in direct comm. 13.3.8 - corrections NF & missing clarification added as NOTE that NF Instance ID in the public key certificate as specified in 3GPP TS 33.310 needs to be present for the verification of NF instance ID in CCA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 13.3.0 (new), 13.3.1, 13.3.1.1, 13.3.1.2, 13.3.1.3, 13.3.2.1, 13.3.2.2, 13.3.2.4, 13.3.8, 13.3.8.1, 13.3.8.2, 13.3.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S3-201798 |  |  |  |  |  |  |  |  |

************ START OF CHANGES

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RG	5G Residential Gateway

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

CCA	Client Credentials Assertion

Cell-ID	Cell Identity as used in TS 38.331 [22]

CHO	Conditional Handover

CIoT	Cellular Internet of Things

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

CP	Control Plane

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EDT	Early Data Transmission

EMSK	Extended Master Session Key

EPS	Evolved Packet System

FN-RG	Fixed Network RG

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPUPS	Inter-PLMN UP Security

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LI	Lawful Intercept

MN	Master Node

MO-EDT	Mobile Originated Early Data Transmission

MT-EDT	Mobile Terminated Early Data Transmission

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G-Capable

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

PUR	Preconfigured Uplink Resource

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SCP	Service Communication Proxy

NOTE: Void.	Security Gateway

SEPP	Security Edge Protection Proxy

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

TSC	Time Sensitive Communication

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

************ NEXT CHANGE

## 13.3	Authentication and static authorization

### 13.3.0	General

This clause describes authentication and static authorization between NFs and NRF (clause 13.3.1), between NFs (clause 13.3.2), between SEPP and NFs (clause 13.3.3), between SEPPs (clause 13.3.4), between SEPP and SCP (clause 13.3.5), between SCP and NFs (clause 13.3.6), and between SCPs (clause 13.3.7). Clause 13.3.8 introduces client credentials assertion (CCA), a concept that enables a NF to authenticate towards the receiving end point.

### 13.3.1	Authentication and authorization between NFs and NRF

#### 13.3.1.1	Direct communication

NF and NRF shall authenticate each other during discovery, registration, and access token request.

In direct communication (model B), NF and NRF shall use one of the following methods for authentication:

-	If the PLMN uses protection at the transport layer as described in clause 13.1, authentication provided by the transport layer protection solution shall be used for mutual authentication of the NRF and NF.

-	If the PLMN does not use protection at the transport layer, mutual authentication of NRF and NF may be implicit by NDS/IP or physical security (see clause 13.1).

#### 13.3.1.2	Indirect communication

In indirect communication, NF and NRF shall use one of the following methods for authentication:

-	Mutual authentication between NF Service Consumer and NRF for discovery (model C) provided by the transport layer protection solution.

-	Client credentials assertion (CCA) and authentication as specified in clause 13.3.8 for model C and model D.

NOTE 1:	CCA based authentication does not provide authentication of the NRF towards the NF Service Consumer since it is not mutually authenticated. It also does not provide protection of the service request sent by the NF Service Consumer to the NRF since no integrity protection is applied as explained in 13.3.8. CCA based authentication is based on generating and sending a JWT token by the NF Service Consumer to the NRF via an intermediate such as the SCP and by this providing e2e authentication of NF Service Consumer to either NRF or NF Service Producer, depending on the deployment model.

-	Implicit authentication, i.e. by relying on authentication between NF Service Consumer and SCP, and between SCP and NRF, provided by the hop-by-hop security protection at the transport layer, NDS/IP, or physical security.

NOTE 2:	Mutual authentication between NF Service Consumer and NRF is not achieved with hop-by-hop security (model D).

NOTE 3:	If only hop-by-hop security is used in a PLMN, the NRF is not able to verify that an access token request sent by SCP on behalf of a certain NF Service Consumer, is actually authorized by this consumer.

#### 13.3.1.3	Authorization of discovery request and error handling

When NRF receives message from unauthenticated NF, NRF shall support error handling, and may send back an error message. The same procedure shall be applied vice versa.

After successful authentication between NRF and NF, the NRF shall decide whether the NF is authorized to perform discovery and registration.

In the non-roaming scenario, the NRF authorizes the Nnrf_NFDiscovery_Request based on the profile of the expected NF/NF service and the type of the NF Service Consumer, as described in clause 4.17.4 of TS23.502 [8].

In the roaming scenario, the NRF of the NF Service Provider shall authorize the Nnrf_NFDiscovery_Request based on the profile of the expected NF/NF Service, the type of the NF Service Consumer and the serving network ID.

If the NRF finds NF Service Consumer is not allowed to discover the expected NF instances(s) as described in clause 4.17.4 of TS 23.502[8], NRF shall support error handling, and may send back an error message.

NOTE 1: 	void.

### 13.3.2	Authentication and authorization between network functions

#### 13.3.2.1	Direct communication

In direct communication (model A and model B), authentication between network functions within one PLMN shall use one of the following methods:

-	If the PLMN uses protection at the transport layer , authentication provided by the transport layer protection solution shall be used for authentication between NFs.

-	If the PLMN does not use protection at the transport layer, authentication between NFs within one PLMN may be implicit by NDS/IP or physical security.

If the PLMN uses token-based authorization, i.e. requesting first NRF (model B) for an authorization token before communicating with another NF, the network shall use protection at the transport layer.

NOTE: Protection method details are described in clause 13.1

#### 13.3.2.2	Indirect communication

In indirect communication scenarios (model C and model D), the NF Service Producer and NF Service Consumer shall use implicit authentication by relying on authentication between NF Service Consumer and SCP, and between SCP and NF Service Producer, provided by the transport layer protection solution, NDS/IP, or physical security.

NOTE 0: Mutual authentication between NF Service Consumer and NF Service Producer is not achieved with hop-by-hop security.

If the PLMN uses OAuth 2.0 token-based authorization as specified by clause 13.4.1.2 and the PLMN's policy mandates that the NRF authenticates the NF Service Consumer before granting a JWT access token, then the NRF indicates to the NF Service Producer in the issued JWT access token that the NF Service Consumer has been authenticated by NRF.

If additional authentication of the NF Service Consumer is required, the NF Service Producer authenticates the NF Service Consumer at the application layer using CCA based authentication as specified in clause 13.3.8.

The NF service consumer authentication based on CCA based authentication is optional to use, and based on operator policy.

#### 13.3.2.3	Inter-PLMN NF to NF communication

NOTE 1: Void

NOTE 2: Void

The present document does not provide a standardised solution for binding 5G SBA REST Service Operations between the PLMN V-SMF and H-SMF over N16 / N32 to GTP-U over N9 in roaming scenarios. To prevent injection or spoofing of UP traffic over N9, it is recommended to use a common firewall that can correlate HTTP/2 methods and GTP-U in order to bind and filter out any malicious traffic on N9. Use of a common firewall may place other implementation restrictions (e.g. co-location of SMF, SEPP and UPF) in order to allow use of a common firewall.

#### 13.3.2.4	Error handling

When an NF receives a message from an unauthenticated NF, the receiving NF shall support error handling, and may send back an error message.

### 13.3.3	Authentication and authorization between SEPP and network functions

NOTE 1: This clause also describes authentication and authorization between SEPP and NRF, because the NRF is a network function.

Authentication between SEPP and network functions within one PLMN shall use one of the following methods:

-	If the PLMN uses protection at the transport layer, authentication provided by the transport layer protection solution shall be used for authentication between SEPP and NFs.

-	If the PLMN does not use protection at the transport layer, authentication between SEPP and NFs within one PLMN may be implicit by NDS/IP or physical security (see clause 13.1).

A network function and the SEPP shall mutually authenticate before the SEPP forwards messages sent by the network function to network functions in other PLMN, and before the SEPP forwards messages sent by other network functions in other PLMN to the network function.

### 13.3.4	Authentication and authorization between SEPPs

Authentication and authorization between SEPPs in different PLMN is defined in clause 13.2.

### 13.3.5	Authentication between SEPP and SCP

Authentication between SEPP and SCP within one PLMN shall use one of the following methods:

-	If the PLMN uses protection at the transport layer, authentication provided by the transport layer protection solution shall be used for authentication between SEPP and SCP.

-	If the PLMN does not use protection at the transport layer, authentication between SEPP and SCP within one PLMN may be implicit by NDS/IP or physical security (see clause 13.1).

A SCP and the SEPP shall mutually authenticate before forwarding incoming or outgoing requests.

### 13.3.6	Authentication and authorization between SCP and network functions

The SCP and network functions shall use one of the following methods described in clause 13.1 to mutually authenticate each other before service layer messages can be exchanged on that interface:

-	If the PLMN uses protection at the transport layer, authentication provided by the transport layer protection solution shall be used for mutual authentication of the SCP and the network functions.

-	If the PLMN does not use protection at the transport layer, mutual authentication of the SCP and network functions may be implicit by NDS/IP or physical security.

Authentication between the SCP and the Network Function may be implicit by co-location.

Editor's Note: Authoriziation between SCP and NFs is ffs.

### 13.3.7	Authentication and authorization between SCPs

SCPs shall use one of the following methods as described in 13.1 to mutually authenticate each other before service layer messages can be exchanged on that interface:

-	If the PLMN uses protection at the transport layer, authentication provided by the transport layer protection solution shall be used for mutual authentication of the SCPs.

-	If the PLMN does not use protection at the transport layer, mutual authentication of the two SCPs may be implicit by NDS/IP or physical security.

Editor's Note: Authorization between SCPs is ffs.

### 13.3.8	Client credentials assertion based authentication

#### 13.3.8.1	General

The Client credentials assertion (CCA) is a token signed by the NF Service Consumer. It enables the NF Service Consumer to authenticate towards the receiving end point (NRF, NF Service Producer) by including the signed token in a service request.

It includes the NF Service Consumer’s NF Instance ID that can be checked against the certificate by the NF Service Producer. The CCA includes a timestamp as basis for restriction of its lifetime.

CCAs are expected to be more short-lived than NRF generated access tokens. So, they can be used in deployments with requirements for tokens with shorter lifetime for NF-NF communication. There is a trade-off that when the lifetime of the CCA is too short, it requires the NF Service Consumer to generate a new CCA for every new service request.

The CCA cannot be used in the roaming case, as the NF Service Producer in the home PLMN will not be able to verify the signature of the NF Service Consumer in the visited PLMN unless cross-certification process is established between the two PLMNs through one of the mechanisms specified in TS 33.310.

CCA does not provide integrity protection on the full service request. Neither does it provide a mechanism for the NF Service Consumer to authenticate the NF Service Producer.

In this clause, CCAs are described generally for both NF-NRF communication and NF-NF communication.

#### 13.3.8.2	Client credentials assertion

CCAs shall be JSON Web Tokens as described in RFC 7519 [44] and are secured with digital signatures based on JSON Web Signature (JWS) as described in RFC 7515 [45].

The CCA shall include:

-	the NF instance ID of the NF Service Consumer (subject);

-	A timestamp (iat) and an expiration time (exp), and

-	The NF type of the expected audience (audience), i.e. the type "NRF", "NF Service Producer", or "NRF" and "NF Service Producer".

The NF Service Consumer shall digitally sign the generated CCA based on its private key as described in RFC 7515 [45]. The signed CCA shall include one of the following fields:

-	the X.509 URL (x5u) to refer to a resource for the X.509 public key certificate or certificate chain used for signing the client authentication assertion, or

-	the X.509 Certificate Chain (x5c) include the X.509 public key certificate or certificate chain used for signing the client authentication assertion.

#### 13.3.8.3	Verification of Client credentials assertion

The verification of the CCA shall be performed by the receiving node, i.e., NRF or NF Service Producer in the following way:

- It validates the signature of the JWS as described in RFC 7515 [45].

- If validates the timestamp (iat) and/or the expiration time (exp) as specified in RFC 7519 [44].

If the receiving node is the NRF, the NRF validates the timestamp (iat) and the expiration time (exp).

If the receiving node is the NF Service Producer, the NF Service Producer validates the expiration time and it may validate the timestamp.

- It checks that the audience claim in the the CCA matches its own type.

- It verifies that the NF instance ID of the NFc in the CCA matches the NF instance ID in the public key certificate used for signing the CCA.

NOTE: NF Instance ID in the public key certificate as specified in 3GPP TS 33.310 needs to be present for the verification of NF instance ID in CCA.

************ END OF CHANGES