3GPP TSG SA WG3 Meeting #100-e	 		S3-201849-r1

e-meeting, 17 – 28 August 2020

Title:	Reply LS on N32-f Protection Policy IE Data-Type Mapping

Response to:	S3-202055 / 5GIS Doc 11_01

Release:	Rel-15

Work Item:	5GS_Ph1-SEC

Source:	SA3

To:	GSMA FASG 5GIS

Cc:	CT4

Contact Person:

#### Name:	Bo Zhang

Tel. Number:	+86 13410752800

E-mail Address:	zhangbo6@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

SA3 would like to thank GSMA FASG 5GIS for their LS on N32-f Protection Policy IE Data-Type Mapping. SA3 has evaluated these IEs and provide the following feedback.

5GIS question 1:

Are these guidelines correct, or are there alternative guidelines which can be given?

SA3 Answer:

Instead of asking whether certain IEs are sensitive or not, GSMA FASG 5GIS could start with the assumption that all the IEs considered by GSMA FASG 5GIS are potentially sensitive. For example, charging IDs and GPSIs should be considered sensitive if not proven otherwise.

Following this general principle, the following concrete answers are given:

| UEID | Please consider charging IDs and GPSI as potentially sensitive, if not proven otherwise. |
| --- | --- |
| LOCATION | Please consider cell-id, physical cell id and TAI as potentially sensitive, if not proven otherwise. |
| KEY_MATERIAL | Please consider KSEAF, encryptionKey, encryptionAlgorithm and UPU related information as sensitive if not proven otherwise. |
| AUTHENTICATION_MATERIAL | Please consider authentication vectors as sensitive. Please consider EAP payload as potentially sensitive if not proven otherwise.. |
| AUTHORIZATION_TOKEN: | Please consider Authorization Tokens as sensitive. |
| OTHER | No comments |
| NON-SENSITIVE | No comments. |

5GIS question 2:

Is it an objective for 3GPP to define the correct Data-Type for IEs?

SA3 Answer:

SA3 considers it is an objective for SA3 to provide guidelines on which IEs need to be protected, and to assist with listing concrete IEs. SA3 does not plan to update TS 33.501 with concrete IEs.

2. Actions:

To GSMA FASG 5GIS group.

ACTION: 	SA3 kindly asks GSMA FASG 5GIS group to take above information into account.

3. Date of Next TSG-SA WG3 Meetings:

SA3#100Bis-e	                	12-16 Oct. 2020				                  eMeeting

SA3#101-e			9-20 Nov. 2020					     eMeeting