3GPP TSG-SA3 Meeting #101-e 	S3-202897

# e-meeting, 9 -20 November 2020													        Revision of S3-20xxxx

# Source:	CATT

# Title:	Update key issue #1 in TR 33.839

# Document for:	Approval

Agenda Item:	5.8

1	Decision/action requested

It is proposed to update KI#1. SA3 is kindly requested to approve this contribution.

2	References

[1]	3GPP TS 33.839, v0.2.0

3	Rationale

The UE ID binding verification requirement was identified and agreed in the last meeting. This contribution proposes to add the requirement to Key issue #1.

4	Detailed proposal

******	BEGINNING OF CHANGES *********

5.1	Key issue #1: Authentication and Authorization between EEC and EES

5.1.1	Key Issue Details

As per TS 23.558 [2], EDGE-1 reference point enables interactions between the Edge Enabler Server and the Edge Enabler Client. EDGE-1 reference point supports registration and de-registration of the Edge Enabler Client to the Edge Enabler Server, retrieval and provisioning of Edge Application Server configuration information; and discovery of Edge Application Servers available in the Edge Data Network.

Edge Enabler server provides functionalities to Edge Enabler client over EDGE-1 reference point such as provisioning of configuration information to Edge enabler client and support the functionalities of application context transfer.

Edge Enabler Client performs the functionalities like configuration information retrieval from the edge enabler server and discovering of the edge application servers available in Edge Data Network. The Edge Data Network is a local Data Network. Edge Application Server(s) and the Edge Enabler Server are contained within the EDN.

The UE is initially provisioned with the configurations required to connect to the Edge Data Network. Upon initial provisioning, the Edge Enabler Client of the UE registers with the selected Edge Enabler Server(s) from the list of provisioned Edge Enabler Server(s). Edge Enabler Client consumes service offered by the Edge Enabler Server, e.g. discovering Edge Application Servers in an area of interest. The procedure enables initialization or update of the Edge Enabler Client context information at the Edge Enabler Server. The Edge Enabler Client sends Edge Enabler Client registration request to the Edge Enabler Server. Edge Application Server discovery enables Edge Enabler Clients to obtain information about available Edge Application Servers of interest. The identification of the Edge Application Servers is based on matching query filters or Application Client Profiles provided in the request.

For the UE identifier, the operator or the third party may allocate a GPSI as specified in TS 23.501 and TS 23.003 to the UE. As specified in TS 23.558, a new edge enabler layer is defined. In order to identify the UE's Edge Enabler Client, the UE uses Edge Enabler client ID as the client identifier at the edge enabler layer. And the Edge Enabler client ID may be used along with GPSI. Then the EEC uses two different identifiers towards the EES, EEC ID and UE identifier (could be GPSI)). Solutions to this key issue need to clearly state which identifier of the EEC they authenticate.

Editor's Note:	It is FFS whether the EEC ID will be unique across different UEs.

5.1.2	Security Threats

When Registration, Discovery , Deregistration is used without authorization, malicious Edge enabler client receive a list of Services and topology structure within Edge Data Network from Edge Enabler Server discovery response message. Received information can reveal Edge Data Network’s topology (e.g. URI, IP address, number of Edge Application Servers, Application Server Functionalities, API type, protocols). Malicious Edge Enabler Client may use this information to launch attacks on Edge Data Network or use this information for competitive reasons.

Editor's Note:	It is FFS how the EEC can be authorized to use edge computing service from 5GS, especially only with EEC ID.

5.1.3	Potential Security Requirements

Edge Enabler Server shall be able to provide mutual authentication with Edge Enabler Client over EDGE-1 Interface.

Edge Enabler Server shall be able to determine whether Edge Enabling client is authorized to access Edge Enabling Server’s services.

*********END OF CHANGES*********