# 3GPP TSG-SA3 Meeting #101-e 	draft_S3-202920-r1

# e-meeting, 9 – 20 November 2020

# Source:	ZTE

# Title:	New solution for key issue#1 in TR 33.850

# Document for:	Approval

Agenda Item:	5.11

1	Decision/action requested

This contribution proposes a new solution for key issue#1 .

2	References

(Reference - in list form - should be made to previous related SA3/3GPP/etc. documents.)

[1]    3GPP TR 33.850 “	Study on security aspects of enhancements for 5G Multicast-Broadcast Services (MBS)”.

3	Rationale

This contribution proposes a new solution for key issue#1.

4	Detailed proposal

**** START OF CHANGES ****

6.X	Solution #X: Authentication and authorization for multicast communication service based on AKMA

6.X.1	Solution overview

This solution, which is based on AKMA, addresses the key issue #1 Security of authentication and authorization for multicast communication service.

- X.2	Solution details

Figure 6.x.2-1 Authentication between the MBSF/MSF-C and UE based AKMA

- UE shall generate the AKMA Anchor Key (KAKMA) and the A-KID from the KAUSF before initiating communication with an AKMA Application Function, i.e MBSF/MSF-C, as specified in TS 33.535[x].

- UE derive a key KMBS for authentication with the MBSF/MSF-C .

- When UE try to join the multicast service, UE computes MAC-I and then UE sends a MBS service request to MBSF/MSF-C. The service request include A-KID and MAC-I.

Editor’s Note: How to derive the MAC-I is FFS.

3-6. Upon receiving the request, the MBSF/MSF-U discovers the AAnF, then AAnF generates KMBS and sends the KMBS to MBSF/MSF-C.

Editor’s Note: It is FFS how to discover the correct AAnF.

Editor’s Note: How MBSF/MSF-C obtains authorization information is FFS.

Editor’s Note: It is ffs whether primary authentication is sufficient to authenticate the UE towards the MBSF.

7.  The MBSF/MSF-C verifies the MAC-I using the KMBS, when the verification is succeed, and if the UE is authorized to perform the operation,Then the MBSF/MSF-C sends a service response to the UE.

6.X.3	Solution evaluation

TBD

**** END OF CHANGES ****

**** START OF CHANGES ****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.757: " Study on architectural enhancements for 5G multicast-broadcast services ".

[3]	3GPP TS 33.246: " Security of Multimedia Broadcast/Multicast Service (MBMS) ".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[x]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

**** END OF CHANGES ****