3GPP TSG-SA3 Meeting #101-e 	S3-202941

# e-meeting, 09-20 November 2020					Revision of S3-XXXX

# Source:	Intel

# Title:	NAS Based Solution for Busy Indication

# Document for:	Approval

Agenda Item:	5.19

# 1	Decision/action requested

It is proposed to approve the Solution for busy indication in MUSIM TR 33XXX.

# 2	References

[1]	3GPP TR 23.761: " Study on system enablers for devices having multiple Universal Subscriber Identity Modules (USIM)"

# 3	Rationale

pCR Proposes a new NAS Based solution related to the security of Busy indication.

# 4	Detailed proposal

****START OF CHANGES ***

## 6.Y	Solution #Y: NAS Security-Based Solution for Busy Indication

### 6.Y.1	Introduction

This Solution relates the KI#XX and proposes a solution to handling the response to paging for MT service. This Solution proposes a solution allowing the UE to send a busy indication to the network in a NAS message as a response to a page.

### 6.Y.2	Solution details

### Figure 6.Y.2-1: NAS Based Security solution for Busy Indication: UE-2 in Idle; UE-1 in Connected

### The procedure below assumes that UE-1 can periodically pause the RRC-connection allowing UE-2 to perform page monitoring.

0.	A device with USIM, i.e., UE1, is in connected mode and UE2 is in IDLE mode.

1.	UE-1, while connected to Network A, goes into absence time to facilitated UE-2 to monitor a scheduled paging occasion and send a busy indication response.

2.	The AMF-2 serving the UE-2 sends a paging request message to RAN-2. RAN-2 pages UE-2

3.  Upon receiving the paging message UE-2, if it decides to send a busy indication, responds with a NAS service request procedure with new cause value "Busy" after RACH procedure. RAN-2 forwards the NAS Service Request message to the AMF-2.

4.	The AMF, upon receiving the busy indication, accepts the NAS service request and suspends paging to UE-2. A new 5G-GUTI is allocated to UE-2, and AMF-2 sends new 5G-GUTI in NAS service accept as a payload.

Editor’s Note: How the UE acknowledges the reception of the new GUTI is FFS.

### 6.Y.3	System impact

UE:

-	Support sending a busy indication.

AMF:

-	Support receiving a busy indication as a response to the N2 paging request message sent to RAN.

### 6.Y.4	Evaluation

## Editor's Note: Each Solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

****END OF CHANGES ***