3GPP TSG-SA3 Meeting #102Bis-e 	draft_S3-211084-r2

# e-meeting, 1 – 5 March 2021						Revision of S3-21xxxx

# Source:	Nokia, Nokia Shanghai Bell

# Title:	KI on Modification and revocation of user consent

# Document for:	Approval

Agenda Item:	2.14

# 1	Decision/action requested

New KI on revocation of user consent.

# 2	References

[1] 3GPP TR 23.700-91: “Study on enablers for network automation for the 5G System (5GS); Phase 2”

# 3	Rationale

A KI on modification and revocation of user consent related to aspects mentioned in 3GPP TR 23.700-91 [1] is proposed.

SA2 concluded in [1] under Key Issue #15: User consent for UE data collection/analysis the following

It is recommended for Key Issue #15 to use the concepts and procedures described in below as the bases for the normative work with further alignments based on the SA WG3 feedback:

a)	UDR (via UDM services) holds the user consent for user related data which is provisioned by MNO as a user subscription information.

b)	A service provider (external to MNO domain) may use NEF parameterProvision_Update service to update or to revoke the user consent to the UDM/ UDR (when applicable).

c)	Alternatively, the user consent parameter may be configured by MNO in the UDM/UDR (when applicable).

d)	Another option is the ASP configures the ASP's Application in UE to provide input data only if user consent is obtained.

e)	A functionality to be provided to support tracking the distribution and usage of any user related information that may be subject to user consent.

NOTE 1:	Whether the functionality described in bullet item e) is hosted by NWDAF, DCCF (and possible interactions with DRF) or as a standalone NF is decided in normative phase in alignment with SA WG3 feedback.

f)	The functionality described in bullet item e) subscribes to the UDM/UDR to receive notifications for any changes in the user consent.

g)	If a requested data is subject to user consent, the NWDAF and/or the functionality described in e) may check the user consent from the UDM/UDR before the data collection.

NOTE 2:	This does not preclude that also other entities (e.g. AMF or SMF) to check availability of user consent with UDM/UDR.

h)	User consent for the user data provided by a data source to a data consumer may have a validity time. On expiry of this validity time, the user data should be either deleted or the user consent for this data to be renegotiated.

i)	(Subject to applicability of Data Protection Regulations) in case of user consent revocation at UDM/UDR, NWDAF deletes the related user data and terminates the data collection from the corresponding source NF.

j)	(Subject to applicability of Data Protection Regulations) in case of user consent revocation (at UDM/UDR), the functionality described in bullet item e) may send a request to NWDAF service consumer(s) to delete the related user data/ analytics.

k)	(Subject to applicability of Data Protection Regulations) in case of user consent revocation (at UDM/UDR), NWDAF service consumer(s) may delete the related user data/analytics on request (refer to bullet item j) or on expiry of validity time (refer to bullet item h).

This key issue focus is on modification and revocation of user consent.

# 4	Detailed proposal

********** START OF CHANGES

### 6.X	Key Issue #X: Modification or revocation of user consent

#### 6.X.1	Introduction

UDR (via UDM services) holds the user consent for user related data which is provisioned by MNO as a user subscription information. A service provider (external to MNO domain) can use Nnef_parameterProvision_Update service to update or to revoke the user consent to the UDM/UDR (when applicable).

#### 6.X.2	Security threats

If user consent modification or revocation is done by an unauthorized party, a service to a consumer can be denied; or service might be granted to the consumer that should not have access to the user data.

#### 6.X.3	Potential security requirements

Editor's Note: Requirements to handle revocation and modification of user consent are FFS.

********** END OF CHANGES