3GPP TSG-SA3 Meeting #103-e	S3-211605

# e-meeting, 17 – 28 May 2021													        	Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	pCR – New KI on Slice authorization under UE quota constraint

# Document for:	Approval

Agenda Item:	5.21

# 1	Decision/action requested

Approve the new KI proposal to eNS2 TR33.874

# 2	References

[1]	23.700-40 Study on enhancement of network slicing; Phase 2

[2]	23.502

# 3	Rationale

The contribution propose a new key issue on Slice authorization under UE quota constraint. This is inline with the study and conclusions in SA2 [1-2].

# 4	Detailed proposal

pCR

***	BEGINNING OF 1st CHANGES ***

## 5.X	Key Issue #X: Slice authorization under UE quota constraint

### 5.X.1	Key issue details

It has been concluded in the study report TR23.700-40 [5] that specifications are to be updated to

-	Define a new network function (NF) to support the storing of network slice information (i.e., the maximum number of UEs and the maximum number of PDU Sessions allowed to be served by a network slice), and managing, updating and enforcing the network slice information for the purpose of network slice access control

In other words, whether a UE is authorized to access a network slice is also subject to quota information, which is assumed to be available in the core network. A general procedure has recently been agreed and included in TS23.502 [3]. However, more details affecting the authroization procedure or the procedure to enforce the quota-constraint authorization has not been updated in TS 33.501 [x1]. In addition, the authorization procedure in roaming scenarios is also yet to be defined.

### 5.X.2	Security threats

If quota information is not taken into consideration to authorize a UE to access a network slice, the network resources will be depleted when more UEs are accessing the network slices. It may cause denial of services (DoS) to all UEs accessing or attempting to access the network slice. On the other hands, the network resources will be wasted if it is over provisioned.

### 5.X.3	Potential security requirements

The 5G system shall support slice authorization enforcing UE quota constraint to prevent denial of services.

***	END OF 1st CHANGES	***

***	BEGINNING OF 2nd CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 23.501: “System architecture for the 5G System (5GS)”

[3] 	3GPP TS 23.502: “Procedures for the 5G System (5GS)”

[4] 	3GPP TS 23.503: “Policy and charging control framework for the 5G System (5GS); Stage 2”

[5] 	3GPP TR 23.700-40: “Study on enhancement of network slicing; Phase 2”

[6] 	3GPP TR 38.832: “Study on enhancement of Radio Access Network (RAN) slicing”

[x1] 		3GPP TS 33.501: "Security architecture and procedures for 5G system".

***	END OF 2nd CHANGES	***