3GPP TSG SA WG 3 Meeting SA3#103e		S3-211375-r4

Electronic meeting, Online, 17 - 28 May 2021

Title:	[draft] Reply LS on Storage of KAUSF

Response to:	LS S3-211375/ C1-211518 on Reply LS on Storage of KAUSF from CT1

Source:	SA3

To:	CT1

Cc:	CT4

Contact person:	Rajavelsamy Rajadurai

Rajvel@]Samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	draft_S3-212103-ry, draft_S3-212104-rx

# 1	Overall description

SA3 thanks CT1 for their reply LS on Storage of KAUSF (S3-211375/C1-211518).

Based on the answer for CT1(S3-211375/C1-211518), SA3 has agreed  the attached Rel-17 CR (draft_S3-212104-rx) to TS 33.501, to mandate the AMF to initiate a security mode control (SMC) procedure soon after a successful primary authentication and key agreement procedure to take the new partial native 5G NAS security context into use. Mandating the AMF to initiate a SMC procedure as soon as possible after a successful primary authentication is applicable only when using 5G AKA based authentication and not for EAP based authentication.

SA3 discussed addressing the potential KAUSF de-synchronization issue from Release 16 as agreed for Release 17 (only after identifying that the primary (re)authentication is successful in the network side, the UE shall store the KAUSF, SOR counter and UE parameter update counter). SA3 requests CT1 to consider the possibility of mandating the SMC procedure soon after a successful 5G AKA authentication procedure from Release 16.

Also, SA3 would like to request CT1, to provide their feedback on the following SA3 agreed solution to address the potential KAUSF de-synchronization issue, specifically when UE receives multiple authentication requests simultaneously. SA3 assumes that CT1 will specify further details (stage-3 aspects) of the following solution.

If UE receives more than one authentication requests via different access types simultaneously (e.g., initial registration after UE powers on, UE initiate the service request procedures simultaneously via both NAS connections), the UE should process the authentication challenges in sequence. The UE should respond to the second authentication challenge after completion of the first authentication procedure. In case if the first authentication procedure is 5G-AKA, then after sending the Authentication Response message to the network the UE should respond to the second authentication challenge. In case if the first authentication procedure is EAP-AKA', then after receiving EAP Success/Failure from the network the UE should respond to the second authentication challenge.

# 2	Actions

To CT1

ACTION: 	SA3 kindly requests CT1,

1. To take the attached SA3 CRs into account when specifying the stage-3 aspects.

2. To consider the feasibility of mandating the AMF to initiate a SMC procedure soon after a successful 5G AKA authentication in Release 16 and provide their decision to SA3.

3. To review and provide their feedback on the  solution to address the potential KAUSF de-synchronization issue, specifically when UE receives multiple authentication requests simultaneously.

SA3 kindly requests CT4 to take attached SA3 CRs into account.

# 3	Dates of next TSG SA WG 3 meetings

Please see the SA3 link from the 3GPP calendar page at  https://portal.3gpp.org/Home.aspx?tbid=386&SubTB=386#/