3GPP TSG-SA3 Meeting #103-e	S3-211609r2

# e-meeting, 17 – 28 May 2021													       	Merger of S3-211609 and S3-211488

# Source:	Huawei, HiSilicon, Interdigital

# Title:	pCR – Conclusions for KI#2

# Document for:	Approval

Agenda Item:	5.7 FS_UAS_SEC

# 1	Decision/action requested

Approve the proposed conclusions to KI#2 for TR33.854

# 2	References

[1]

# 3	Rationale

This contribution proposes to conclude the KI#2 study and starts normative work

(KI#2: Pairing authorization for UAV and UAVC)

# 4	Detailed proposal

pCR

***	BEGINNING OF 1st CHANGES    ***

## 7.2	Conclusions for KI#2

Pairing Authorization for UAV and UAVC is recommended for the normative work based on the following solutions and principles:

- Pairing authorization is performed after successful UAA between UAV and USS/UTM

- Pairing authorization is performed either during registration or PDU session establishment/modification procedure (5G solution #5, #11, #14, #15 as base) and enforced in the 3GPP network based on connectivity  information received from USS.

- Both SMF and authorized USS/UTM may trigger pairing authorization. Authorized USS/UTM may trigger updating and revocation of pairing authorization (sol#15 as base for UAV-C change)

- For EPS: solution #13 is chosen as the basis for normative work, with similar principles as for 5GS above.

- During pairing authorization procedure, CAA Level UAV ID and 3GPP UAV ID are used to identify UAV.

- The messages used for pairing authorization that are exchanged between UAV and USS/UTM are included in transparent containers and the content is out of scope of 3GPP,

***	END OF 1st CHANGES	***