3GPP TSG-SA3 Meeting #103-e 	S3-212124-r12

e-meeting, 17 - 28 May 2021

Title:	LS on Clarifications of Network slice selection during AMF Reallocation

Response to:	N/A

Release:	Rel. 17

Work Item:	FS_AMFREAL_SEC

Source:	Lenovo, Motorola Mobility to be SA3

To:	SA2

Cc:

Contact person:	Sheeba Backia Mary Baskaran

smary@lenovo.com

017411977172

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TR 33.864

# 1	Overall description

## 1.1	General

SA3 is currently working on the "Study on the security of Access and Mobility Management Function (AMF) re-allocation" in TR 33.864. The study is focusing on addressing the registration failure issue related to AMF re-allocation and indirect reroute via RAN (option (B)) specified in TS 23.502 Clause 4.2.2.2.3. TR 33.864 Key Issue #1 describes the registration failure scenario in detail.

To solve the above repeated registration failure issue, SA3 is discussing solutions 6 and 7 from the TR 33.864.

In the procedure of registration with AMF reallocation via RAN defined by SA2, the initial AMF may only be able to obtain Requested S-NSSAIs by one of two methods for different scenarios mentioned in 33.864 Clause 4.3 i.e., (method 1) Requested S-NSSAIs can be obtained after NAS SMC procedure, i.e. from the complete Registration Request message in NAS Security Mode Complete message (or) (ii) if the UE sends a protected registration request, then the initial AMF if it can fetch current 5G security context from the source/old AMF, then the initial AMF will be able to get the requested NSSAI without NAS SMC. For scenarios that require method-1, Solution 6, 7 in TR 33.864 however proposes not to run NAS SMC, which means that, the initial AMF may not have the Requested S-NSSAIs. Solution 6, and 7 have the following points as their core principle, which need to be evaluated by SA2:

1. The initial AMF uses the Requested NSSAI (if available to the AMF) and subscribed NSSAI along with existing information/inputs for Network slice selection using Nnssf_NSSelection_Get service operation defined in TS 23.502 clause 5.2.16.2.1.

2. If the Requested NSSAI is not available to Initial-AMF, the initial AMF does not performs NAS SMC to fetch Requested NSSAI (contained in, e.g. Full Registration Request), and performs Nnssf_NSSelection_Get service operation defined in TS 23.502 clause 5.2.16.2.1 with subscribed NSSAI along with existing information/inputs for Network slice selection.

3. If the initial AMF determines that a reroute via RAN is required, the initial AMF skips NAS SMC with UE:

The CT1 TS 24.501, clause 5.4.1.2 "EAP based primary authentication and key agreement procedure, states the following: If the authentication of the UE completes successfully and the serving AMF does not intend to initiate a security mode control procedure bringing into use the partial native 5G NAS security context created by the EAP based primary authentication and key agreement procedure, then the EAP-success message, and the ngKSI are transported from the network to the UE using the AUTHENTICATION RESULT message of the EAP result message transport procedure.

NOTE 1:	The serving AMF will not initiate a security mode control procedure after the EAP based primary authentication and key agreement procedure e.g. in case of AMF relocation during registration procedure.

## 1.2	Questions

Based on the above information SA3 would like to know the views of SA2 for the following questions respectively.

- Question 1: The Initial AMF during registration procedure after successful ‘primary authentication’ does not initiate NAS SMC, and hence may not obtain the requested S-NSSAIs. If the initial AMF successfully obtains ‘slice selection subscription data’ from the UDM with SUPI, is it feasible for the initial AMF to perform network slice selection using the Nnssf_NSSelection_Get service operation without Requested NSSAI, but using all other existing IEs as inputs (e.g., subscribed NSSAI etc. as in 23.502 clause 5.2.16.2.1)?

- Question 2: Is it feasible to use the TS 23.502 Clause 5.2.16.2.1 Nnssf_NSSelection_Get service operation for network slice selection during registration procedure?

- Question 3: Can the AMF skip NAS SMC for retrieving the Full Registration Request containing Requested-NSSAI before performing the Nnssf_NSSelection_Get service operation?

- Question 4: Can the initial AMF determine that NAS reroute is needed without Requested NSSAI?

- Question 5: How can target AMF obtain the Requested NSSAI? After the target AMF receives the Requested NSSAI, is it possible that AMF reallocation will occur?

- Question 6: Can solutions 6 and 7 meet the SA2 working principles for the AMF re-allocation and reroute via RAN and related network slice selection principles for registration?

# 2	Actions

To SA2:

ACTION: 	3GPP TSG SA WG3 kindly asks SA2 to answer the above questions.

# 3	Dates of next TSG SA WG 3 meetings

SA3#103Bis-e	5 - 9 ~July 2021		Electronic meeting (TBC)

SA3#104-e	16 - 27 August 2021	Electronic meeting