# 3GPP TSG SA WG3 Meeting 103-e	S3-212257r12

# Electronic meeting, 17-28 May 2021

Title:	Reply LS on Small data transmission

Response to:	LS S3-211426 (R2-2104401) on Small data transmissions

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	SA3

To:	RAN2

Cc:	SA2

Contact person:	Alec Brusilovsky

Alec.Brusilovsky@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

SA3 would like to thank RAN2 for their LS on Small data transmission.

RAN2 asked the following questions:

Q1:  Can a CCCH message reusing the I-RNTI and resumeMAC-I be transmitted again in the same cell after SDT initiation, e.g., similar to legacy RRC Reject case (but without having received RRC Reject at the UE)?.

Q2: Can NCC and I-RNTI from a former cell in which an SDT procedure was initiated be reused to initiate a new SDT procedure in a new cell?

SA3 would like to acknowledge the security issues related to reusing the same I-RNTI and NCC with the same cell scenario or mobility scenarios as cell reselection.

However, for both cases (same cell and different cell), SA3 would like to provide the following feedback.

SA3 would like to point out that to avoid replay attacks, keystreams should not be reused. The inputs of keystreams include the following input parameters: KEY, COUNT (e.g., PDCP count), MESSAGE, DIRECTION, and BEARER. Any change in an input parameter will result in a different keystream.

SA3 asks RAN2 to confirm that the above requirements are met. The above requirements are strict and cannot be overruled. SA3 asks RAN2 to verify that the above requirements are met.

During the SA3#103-e discussion, SA3 could not agree on whether these issues represent “corner” or “normal” cases. For that, SA3 need to receive a feedback from RAN2 on the abrupt SDT termination and scenario(s) under which it can happen.

Q1: Under which scenarios RAN2 see abrupt SDT termination as a normal scenario?

# 2	Actions

To RAN2, SA2

ACTION: 	3GPP TSG SA WG3 would like RAN2 and SA2 to take the above feedback into account and answer the above question.

# 3	Dates of next TSG CT WG1 meetings

TSG SA WG3 Meeting 104-e (TBC)	16-27 August 2021