# 3GPP TSG SA WG3 Meeting 103-e	S3-212257

# Electronic meeting, 17-28 May 2021

Title:	Reply LS on Small data transmission

Response to:	LS S3-211426 (R2-2104401) on Small data transmissions

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	SA3

To:	RAN2

Cc:	SA2

Contact person:	Alec Brusilovsky

Alec.Brusilovsky@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1	Overall description

SA3 would like to thank RAN2 for their LS on Small data transmission.

RAN2 asked following two questions.

Q1:  Can a CCCH message reusing the I-RNTI and resumeMAC-I be transmitted again in the same cell after SDT initiation, e.g. similar to legacy RRC Reject case (but without having received RRC Reject at the UE)?.

Q2: Can NCC and I-RNTI from a former cell in which an SDT procedure was initiated be reused to initiate a new SDT procedure in a new cell?.

For both cases (same cell, and different cell), SA3 would like to give following feedback.

SA3 requires that keystreams are never reused. The inputs of keystreams include the following parameters: KEY, COUNT, MESSAGE, DIRECTION and BEARER. One of the inputs changes will result in different key stream.

SA3 asks RAN2 to confirm that the above requirements are met. The above requirements are strict and cannot be overruled.

SA3 asks RAN2 to discuss any potential solution and make an assessment if the complexity of such solution will commensurate with the risk. If risk is considered too low, then a note has to be recorded that 3GPP made a conscious decision for availability to accept the low risk and not provide any solution.

Further questions to RAN2: (a) Is RAN2 thinking of allowing “two and only two” RRCResumeRequests or will there be more? (b) Will there be separate “cause” values in RRCResumeRequest for SDT and non-SDT?

2	Actions

To RAN2, SA2

ACTION: 	3GPP TSG SA WG3 would like RAN2 and SA2 to take the above feedback into account.

3	Dates of next TSG CT WG1 meetings

TSG SA WG3 Meeting 103-Bis-e (TBC)	05-09 July 2021

TSG SA WG3 Meeting 104-e (TBC)	16-27 August 2021