3GPP TSG-SA3 Meeting #103-e 	S3-211397

# e-meeting, 17 - 28 May 2021

Source:	3GPP SA2

# Title:	Reply LS on Secondary AUTH for 5GS interworking with EPS

# Document for:	Information, Discussion

Agenda Item:  3

3GPP TSG SA WG2 Meeting SA2#143E 		S2-2101305

Feb 24 – Mar 09, 2021, Electronic, Elbonia

Title:	Reply LS on Secondary AUTH for 5GS interworking with EPS

Response to:	LS on Secondary AUTH for 5GS interworking with EPS (S2-2100057/C3-210377)

Release:	Rel-17

Work Item:	5GS_Ph1

Source:	SA2

To:	CT3

Cc:	SA3

Contact person: 	Stefan Rommer

stefan.rommer@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.502 CR2475 (Rel.16)

# 1	Overall description

SA2 would like to thank CT3 for the LS on Secondary AUTH for 5GS interworking with EPS.

SA2 has discussed the questions and would like to provide the following replies:

CT3 Q1: Whether EAP based secondary authorization/ authentication is also applicable for EPS, when the UE supports EAP.

SA2 reply: EAP based secondary authorization/ authentication has only been defined for 5GS and is thus not applicable to EPS in existing releases. SA2 expects that in case EAP based secondary authorization/ authentication is to be introduced in EPS it would require a new work item in SA2.

CT3 Q2: When the DN-AAA server initiates EAP based re-authorization but UE has moved from 5GS to EPS, whether such re-authorization will be supported.

SA2 reply: If the re-authorization is associated with EAP based re-authentication procedure, then the re-authorization will not be supported since EAP-based re-authentication cannot be performed when the UE is in EPS in existing releases. However, if based on local policy the DN-AAA server initiates DN-AAA re-authorization without performing re-authentication, then a DN-AAA re-authorization (without EAP-based re-authentication) can be supported even when UE is in EPS: this may be used. to provide new parameters from the DN-AAA server to SMF+PGW-C.

CT3 Q3: If only PAP/CHAP based secondary authorization/authentication is applicable in EPS, how to handle the case when the DN-AAA server initiates EAP based re-authorization but UE has moved from 5GS to EPS.

SA2 reply: SA2 assumes that CT3 refers to the re-authorization associated with EAP-based re-authentication procedure scenario. SA2 expects that in such case the SMF+PGW-C, that receives the re-authentication request from the DN-AAA, can inform the DN-AAA server that the UE is not available for EAP-based re-authentication at the moment. The SMF+PGW-C should not initiate PDN connection release: the DN-AAA can decide based on the reply from SMF+PGW-C and based on local policy what actions to take in that case, but this is out of 3GPP scope.

SA2 has agreed the attached CR to clarify the questions above.

# 2	Actions

To CT3

ACTION: 	SA2 kindly requests CT3 to take the above information into account

# 3	Dates of next TSG SA WG2 meetings

SA WG2 Meeting #144E 		April 12-16, 2021

SA WG2 Meeting #145E		May 17-28, 2021