3GPP TSG-SA3 Meeting #103-e	S3-212181

# e-meeting, 17 – 28 May 2021													        	Revision of S3-211606

# Source:	Huawei, HiSilicon, InterDigital

# Title:	pCR – Evaluation of solution #3

# Document for:	Approval

Agenda Item:	5.7 FS_UAS_SEC

# 1	Decision/action requested

Approve the proposed text evaluating solution #3 for TR33.854

# 2	References

[1]

# 3	Rationale

This contribution proposes to add evaluation text to solution #3.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES ***

### 6.3.3	Solution evaluation

This solution is aligned with TR 23.754 [3] conclusions for UAV authentication and authorization by USS/UTM (UUAA) during Registration, including the usage of a generic (API based) procedure using a UAS NF.

This solution fully addresses the requirements of Key Issue #1:

•	The solution uses a generic (i.e., API based) procedure for UUAA during Registration via a Proxy A&A (UAS NF). The UE includes its CAA-level UAV ID to register for UAS services. After a successful primary authentication, the AMF triggers a UUAA if the UE has a valid Aerial subscription and if there is no UUAA ongoing or a valid result from a successful prior UUAA run. The AMF triggers UUAA after sending a Registration Accept message indicating a pending UUAA. The authentication method and content of authentication message used for UUAA are not in 3GPP scope.

•	The solution enables the revocation of UAV authorization by the USS/UTM. The revocation request is received by the UAS NF which notifies the AMF. AMF may trigger a PDU Session release for the relevant PDU Sessions (used for communication USS/UTM and/or for C2 communications) and/or a DeRegistration procedure.

•	Authentication of USS/UTM is handled by the Proxy A&A function by means of provisioned aviation domain certificates.  USS/UTM address may be obtained from UE or from a trusted resolution function which provides a USS/UTM address based on a CAA-level UAV ID.

API based procedure introduces a new mechanism compared to existing EAP framework. NOTE: Usage of API based is used to address an explicit requirement from the UTM community

NOTE: How and whether to protect the transparent containers used for UAV-USS communication during UUAA will be determined during the normative phase

NOTE: IETF/3GPP protocols are readily available for EAP based mechanism to protect the transparent containers.

***	END OF CHANGES	***