3GPP TSG-SA3 Meeting #104-e ad-hoc 	draft_S3-213346-r3

# e-meeting, 27 – 30 September 2021									                    Revision of S3-213346

# Source:	Nokia, Nokia Shanghai Bell, AT&T, Qualcomm Incorporated, Lenovo, Motorola Mobility, Huawei, HiSilicon, Samsung, Ericsson, Intel corporation

# Title:	Conclusion for key issue#1

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed conclusion for key issue #1 of TR 33.881.

# 2	References

[1] TR 33.881

# 3	Rationale

This contribution proposes to add conclusion for Key Issue #1. Few advantages of solution#1 are listed below.

- Maintains the function of the AUSF as the primary EAP Authentication Server (inline with existing TS 33.501 clause 6.1.1.2).

- Introduces a new 5GC Network Function and interface that will be forward compatible with the 5GC.

- 5G NSWO can be deployed without any dependency with 4GC.

# 4	Detailed proposal

************* START OF CHANGES**************

# 7	Conclusions

# 7.1.1   Conclusion for key issue #1

Solution#1 is selected as the basis for the normative work of key issue#1 “Support of EAP-AKA’ authentication for NSWO” with the main charactetistics as follows

- The WLAN access is connected to a new NSWO NF which acts as a proxy of NSWO authentication requests towards the 5GS.

- The UE makes use of a SUCI which is deconcealed by the 5GS in the UDM/SIDF.

************** END OF CHANGES***************