3GPP TSG-SA3 Meeting #104-e ad-hoc 	S3-213579

# e-meeting, 27 - 30 September 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | draftCR | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [33.180] R17 Preconfigured group clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCXSec2 |  |  |  |  |  | Date: |  |  | 2021-09-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The existing method for securing group regroup with preconfigured group and user regroup with preconfigured group uses normal MC group security procedures. A reader not familiar with MC security may find it confusing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add statements that group regroup with preconfigured group security and user regroup with preconfigured group security follow existing group security procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A reader may find it difficult to understand group regroup with preconfigured group security and user regroup with preconfigured group security. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.X (new), 7.3.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********************  START of change 1 **********************

### 7.3.X	Group regroup with preconfigured group

Group regroup with preconfigured group is defined in TS 23.379 [2] and TS 23.280 [36]. The basis of this feature is to allow an authorized MC user to dynamically create a temporary group consisting of a set of predefined groups (i.e. the affiliated members of the predefined groups become members of this new regroup group).  For common group configuration and security, the group regroup with preconfigured group uses a group that is common to all the invited users.  TS 23.379 [2] and TS 23.280 [36] refer to this common group as a preconfigured group.  All users invited to join a secure group regroup with preconfigured group call shall be members of the indicated preconfigured group and shall have obtained the preconfigured group configuration and GMK.

Security for group regroup with preconfigured group follows typical group security establishment procedures using the GMK and GMK-ID obtained through the preconfigured group configuration.  The group configuration to be used (i.e. the GMK and GMK-ID) is identified in the initial preconfigured regroup request message.  The GMK and the GMK-ID from the identified preconfigured group shall be used to derive the master key and master salt as described in clause 7.4.2 and used to secure the media as described in clause 7.5.

### 7.3.Y	User regroup with preconfigured group

User regroup with preconfigured group is defined in TS 23.379 [2] and TS 23.280 [36]. The basis of this feature is to allow an authorized MC user to dynamically create a temporary group consisting of a specified set of users (i.e. the users identified in the user regroup become members of this new temporary group).  For common group configuration and security, the user regroup with preconfigured group uses a group that is common to all the invited users.  TS 23.379 [2] and TS 23.280 [36] refer to this common group as a preconfigured group.  All users invited into a secure user regroup with preconfigured group call shall be members of the indicated preconfigured group and shall have obtained the preconfigured group configuration and GMK.

Security for user regroup with preconfigured group follows typical group security establishment procedures using the GMK and GMK-ID obtained through a preconfigured group configuration.  The group configuration to be used (i.e. the GMK and GMK-ID) is identified in the initial preconfigured regroup request message.  The GMK and the GMK-ID from the identified preconfigured group shall be used to derive the master key and master salt as described in clause 7.4.2 and used to secure the media as described in clause 7.5.

********************  END of change 1 **********************