3GPP TSG-SA3 Meeting #104-e 	S3-212621

# e-meeting, 16 – 27 August 2021, Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0175 | rev | <Rev#> | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security Solution for temporary group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S3 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | MCXSec2 |  |  |  |  |  | Date: |  |  | 2021-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The key management for group communication is based on the GMK which is distributed with the “Notify group configuration request”at group management phase before the group communication happens. However, when it comes to the temporary group call for group regroup or user regroup procedure, the existing security mechanim defined in 3GPP TS 33.180 is not applied as the group call is not over a single group that already formed. The GMK is used to derive the SRTP Master Key/Salt which is used to establish protection of the group-broadcast group media. The security solution is to provide a pre-configured GMK for all the groups or users which may included in the temporary group call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose a security solution for temporary group call for group regroup or user regroup procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no security solution for temporary group call procedure.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change ****************

7.3.X	Temporary group call

Temporary Group call peformed by group regroup is defined in clause 10.6.2.5.3 of 23.379 [2] and Temporary Group call peformed by user regroup is defined in clause 10.6.2.8 of 23.379 [2]. Furthermore, the temporary group call may be initiated based on Group regroup with preconfigured group or User regroup with preconfigured group defined in TS 23.379 [2] and TS 23.280 [36].

Security for temporary group call is based on the GMK and GMK ID obtained through a predefined group security configuration. The GMK and the GMK-ID from predefined group security configuration is used to derive the master key and master salt as described in clause 7.4.2 and used to secure the media as described in clause 7.5 when a call is initiated for temporary group.

- For temporary Group call peformed by group regroup or user regroup, the predefined group security configuration to be used is identified during the temporary group call performed by group regroup or user regroup procedure.

- For temporary Group call based on the Group regroup with preconfigured group or User regroup with preconfigured group, the predefined group security configuration(i.e. the GMK and GMK-ID)  to be used is identified during regroup procedure.

*************** End of the Change ****************