3GPP TSG-SA3 Meeting #104-e 	S3-213150-r4

e-meeting, 16 - 27 August 2021

Title:	Reply to LS on broadcast of NTN GW or gNB position

Response to:	LS S3-212431 (R1-2106332) on broadcast of NTN GW or gNB position from RAN1

Release:	Release 17

Work Item:	TEI17

Source:	Ericsson (to be SA3)

To:	RAN1

Cc:	SA1, SA3-LI

Contact person:	Vlasios Tsiatsis

vlasios dot tsiatsis at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA3 would like to thank RAN1 for the LS on broadcast of NTN GW or gNB position. SA3 would like to answer the questions included in the LS from a security perspective. For regulatory aspects SA1 and SA3-LI may respond.

RAN1 Question 1: Is there any security/regulatory aspect that needs to be taken into account if the NTN-GW/gNB position is broadcasted including any aspects related to accuracy of the position?

SA3 Answer

It should be observed that the broadcast information of the position of a Non-Terrestrial Network (NTN) GW or gNB is unprotected. As a result, the NTN GW or gNB location information cannot be fully trusted as it is subject to tampering. Unprotected broadcast information cannot be trusted in terrestrial networks either.

Exposing the location information of a NTN gNB or GW will increase the attack vector, and lowers the barrier for an attacker.

RAN1 Question 2:  Is there any security/regulatory aspect that needs to be taken into account if the NTN-GW/gNB position is possible to be derived by the UE with assistance information from the network including any aspects related to accuracy of the position?

SA3 Answer

If the UE has the means to calculate the positions of NTN gNBs or GWs then the NTN gNB or GW location may not be easily accessible to a casual user but it can be accessible to a resourceful attacker. As a result the situation is not different from an attacker's point of view compared to the alternative to provide the NTN gNB or GW location information in a broadcast channel.

It can be noted that a UE in a terrestrial network may also calculate the position of the serving gNB by using assistance information from the network after deciphering, namely by means of trilateration of timing advance commands (TAC) received from the gNB. Unprotected assistance information will lowers the barrier for an attacker. Therefore, the potential risk is increasing if any location related information is disclosed.

2	Actions

To TSG RAN WG1

ACTION: 	SA3 asks RAN1 to take the answers to the questions into account.

3	Dates of next TSG SA WG 3 meetings

SA3#105		8 - 12 November 2021	Sophia Antipolis, FR

SA3#106		7 - 11 February 2022		TBC, EU