3GPP TSG-SA3 Meeting #104-e 	S3-212585

# e-meeting, 16 - 27 August 2021													Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	UAA re-authentication procedure (5G)

# Document for:	Approval

Agenda Item:	4.20 ID_UAS

# 1	Decision/action requested

Approve the proposed pCR as normative text

# 2	References

[1]

# 3	Rationale

This contribution proposes the UAA re-authentication procedure based on the agreed principle in the study. It is in-line with SA2’s procedure as well.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES (all text are new)  ***

### X.1.1.4			UUAA re-authentication procedure (5G)

As described in X.1.1.4, the USS or the AMF may initiate the Re-authentication procedure for the UAV at any time. This clause describes the USS initiated Re-authentication procedure (the AMF initiated Re-authentication procedure is described in the clause X.x.x.2). The below description considers only the security related parameters (for full details of the flows see TS 23.256 [aa]).

Figure X.1.1.4-1: UUAA re-authentication in 5GS

1. The USS sends a re-authentication request for the UAV to UAS-NF that includes GPSI.

2. The UAS NF retrieves the UAV UE's UUAA context. The UE’s UUAA context contains the mapping between the GPSI and the USS that performed UAA. The UAV-NF verifies the USS re-authentication request by checking whether the GPSI and the USS match the stored mapping of GPSI and USS. The UAV-NF shall only continue the re-authentication procedures if match.

The UAS NF determines whether the target NF is an AMF or an SMF.

- If the target NF is an AMF, the UAS NF further determines the target AMF for re-authentication and continues step 3a.

- If the target NF is an SMF, the UAS NF further determines the target SMF for re-authentication and continues step 3b.

3a or 3b. The UAS NF sends to either the target AMF or the target SMF the UAA re-authentication request for the UE identified by the GPSI.

4. The UAS NF responses the USS that the UAA Re-authentication has been initiated.

5a. If the target NF is an AMF, the AMF initiates re-authentication of the UAV as described in the clause x.1.1.2 (step 2 to step 9).

5b. If the target NF is an SMF, the SMF initiates re-authentication of the UAV as described in the clause x.1.1.3 (step 2 to step 7).

***	END OF CHANGES   ***