3GPP TSG-SA3 Meeting #104e 	S3-212735

# e-meeting, 16 - 27 August 2021

# Source:	Ericsson

# Title:	Change request to living document: Key hierarchy

# Document for:	Approval

Agenda Item:	4.17

# 1	Decision/action requested

Approve this contribution to be included in the eNPN draft CR for TS 33.501

# 2	References

[1]	-

# 3	Rationale

This document describes the normative text for clause I.2.2.2 "Credentials holder using AAA server for primary authentication" in the eNPN draft CR for TS 33.501.

# 4	Detailed proposal

***	BEGINNING OF CHANGES ***

### I.2.3.2		Credentials holder using AAA server for primary authentication

When running primary authentication towards an external Credentials holder using AAA server for authentication as specified in clause I.2.2.3 the derivation of KAUSF is based on the EAP-method credentials in the UE and AAA-S and shall be done as shown in Figure I.2.3.2-1.

EAP method credentials

EAP

authentication

MSK

K

AUSF

Figure I.2.3.2-1: KAUSF derivation for primary authentication towards an external Credentials holder using AAA server

KAUSF shall be derived by the AUSF and UE from the MSK derived during the EAP authentication as specified in clause I.2.2.3.1.

All of figures 6.2.1-1, 6.2.2.1-1 and 6.2.2.2.2-1 from the KAUSF downwards are used without modification. Similarly, text relating to the key hierarchy, key derivation and key distribution in clauses 6.2.1, 6.2.2.1 and 6.2.2.2 for keys derived from KAUSF (e.g. KSEAF, KAMF, KgNB etc) apply without modification.

***	END OF CHANGES ***