3GPP TSG-SA3 Meeting #104-e 	S3-212593

# e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	A solution with restrictions in EAC inactive modes

# Document for:	Approval

Agenda Item:		5.21 FS_eNS2_SEC

# 1	Decision/action requested

Include the solution in TR33.874

# 2	References

[1]

# 3	Rationale

This contribution proposes a new solutoin to address the KI on DoS to NSAC procedure.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES (all text are new) ***

## 6.Y	Solution #Y: Restrictions in EAC mode

### 5.1.1	Introduction

The newly introduced Early Admission Control (EAC) mode in TS23.501 [2] and TS23.502 [3] should be operated with restrictions in order to prevent potential DoS attacks. This solution proposes to add in a maximum number of UE can be accepted by an AMF before updating NSACF.

### 5.1.2	Solution details

0. Assuming AMF has been configured with a parameter “Nmax” which refers to the maximum of UE that the AMF can admit during the EAC mode not active.

NOTE: When EAC mode is not active, the AMF triggers the Number of UEs per network slice availability check and update procedure after the Registration Accept message is sent as specified in the clause 4.2.1.1.2 in TS23.502.

1. Before sending out the Registraion Accept message, the AMF checks whether the total number of UEs the AMF has admitted is more than Nmax.

If negative, the AMF continues to send out the Registration Accept message as in step 21 of the clause 4.2.2.2.2 in TS23.502.

Otherwise, the AMF triggers the Number of UEs per network slice availability check and update procedure as stated in 4.2.11.2 in TS23.502.

### 5.1.3	Evaluation

TBC

***	END OF CHANGES	***