3GPP TSG-SA3 Meeting #104-e 	S3-212733

# e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

# Source:	Ericsson, Huawei, Interdigital, Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

# Title:	Conclusions for KI#1 (external entity) on SUPI privacy and usage of MSK

# Document for:	Approval,

Agenda Item:	5.12

# 1	Decision/action requested

Approve the pCR below with conclusions for KI#1 (external entity).

# 2	References

[1]	3GPP TR 33.857 "Study on enhanced security support for Non-Public Networks (NPN)"

[2]	draft_S3-211074-r3.doc

# 3	Rationale

This is a resubmission of S3-211767 discussed at SA3#103-e.

This proposed conclusion is based on draft_S3-211074-r3 [2] discussed at SA3#102bis-e. The differences to the draft are the following:

-	The bullet on the optional support of EAP-TTLS with TLS tunnel terminating at AUSF has been removed.

-	The conclusion on re-use of existing SUPI privacy mechanisms has been moved so that it now applies to both the non-5GS aware and the 5GS-aware case.

# 4	Detailed proposal

***	BEGINNING OF CHANGES ***

## 7.1	Conclusions on KI #1: Credentials owned by an external entity

For SUPI privacy, reuse existing mechanism in Annex I.5 in TS 33.501.

In case that the external entity is 5GS aware (i.e., has the AUSF/UDM and is capable of deriving 5G key hierarchy after a successful primary authentication), it is concluded that the existing 5GS roaming architecture is reused.

In case that the external entity is non-5GS aware (legacy AAA server), the following is concluded:

- The SNPN access with a credential owned by an external entity is performed via an AUSF in the SNPN that is enhanced to interface with the external entity.

- When the legacy AAA server supports key deriving EAP method, the enhanced AUSF uses the MSK received from the AAA to derive the necessary 5G keys (e.g., KAUSF, KSEAF).

Editor’s Note: Further conclusion(s) are FFS.

***	END OF CHANGES ***