3GPP TSG-SA3 Meeting #104-e 	S3-212902

# e-meeting, 16 - 27 August 2021, Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0100 | Rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refresh of KAKMA and KAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S3 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  | Date: |  |  | 2021-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Excerpts from 3GPP TS 33.535: 6.4.1 KAKMA re-keying KAKMA shall be re-keyed by running a successful primary authentication as described in clause 6.1. 6.4.2 KAF re-keying The KAF re-keying depends on the lifetime of the KAF and may be trigged by the AF, which means that when a new KAKMA is derived, the KAF will not be re-keyed automatically. When the lifetime of KAF expires, the AF may reject UE’s access to the AF or refresh the KAF as description in clause 6.4.3 based on its policy. If there has been a change of KAUSF (e.g., due to a successful run of primary authentication), the UE may re-try accessing the AF by using the A-KID derived from the new KAUSF. As per the above excerpts, the KAF may not be refreshed after its lifetime expiry, until a new primary authentication takes place. This means a user may not be able to use an application (requiring authentication using AKMA) after the KAF expires and until a new primary authentication procedure takes place (which may happen after a very long period of time). The KAF should be refreshed when needed by the applications that are depending on the KAF and hence a mechanism is required for the AKMA service to request the AUSF to initiate the primary authentication procedure to refresh the KAUSF in order to derive a fresh KAF key, once the lifetime is expired. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes: - Upon KAF lifetime expiry, AF requests AAnF to provide new KAF. - AAnF, upon determining that KAF cannot be refreshed (as same key will get generated), requests AUSF to generate new KAKMA. - Perform a re-authentication procedure as defined in TS 33.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for explict triggering of AKMA key refresh, leads to unavailability of service, if there is no valid KAF exist in the UE and in the AF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3, 6.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of Changes ***

### 6.4.3	KAF refresh

Ua* protocol may support refresh of KAF. If the Ua* protocol supports refresh of KAF, the AF may refresh the KAF at any time using the Ua* protocol.

If Ua* protocol does not support refresh of KAF, KAF cannot be refreshed until next primary authentication and the AF may reject the UE’s access to the application, upon KAF lifetime expiry. Depending on the local policies, the AF may request initiation of fresh primary authentication by following the procedure described in clause 6.6.

*** Next Change ***

### 6.6	Key Update Procedure in AKMA

In case KAF expires, the AF may request initiation of fresh primary authentication using the procedure detailed in figure 6.6-1:

Figure 6.6-1: KAF Refresh Procedure

- UE registers to the network and shall perform the primary authentication. Upon successful completion of primary authentication, if the UE is authorised for AKMA service, then the key KAKMA and A-KID are derived and provided to the AAnF as described in clause 6.1 of this document.  When UE wants to establish application session with the AF, the UE shall generate KAF and request for application session establishment with AF by providing A-KID in the Application session establishment request. AF shall retrieve the KAF and the KAF expiration time from AAnF, by providing AF-Identity (e.g. AF-FQDN) and A-KID.

  - When the UE request the AF for access and if the KAF lifetime expires or is about to expire, the AF shall request the AAnF to refresh the key KAF by sending Naanf_AKMA_ApplicationKey_Get request, including key refresh indicator. If the KAF lifetime expires, the AF shall reject the UE’s access request, as the old A-KID is no longer valid, it shall request the UE to connect using a new A-KID.

  - Upon determining that the KAF cannot be used further, since A-KID associated KAF is already used for the AF, the AAnF shall request the AUSF to generate a fresh KAKMA, by sending Nausf_AKMAKey_Get request.

- Upon receiving the request from the AAnF, the AUSF initiates re-authentication procedure as specified in clause 6.1.2 of TS 33.501.

*** End of Changes ***