3GPP TSG-SA3 Meeting #104-e 	S3-212937

# e-meeting, 16 - 27 August 2021

# Source:	Ericsson

# Title:	Conclusions for KI#10 "Authorization during Edge Data Network change"

# Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

Approve the pCR to TR 33.839 [1] below.

# 2	References

[1]	3GPP TR 33.839 "Study on security aspects of enhancement of support for edge computing in 5G Core (5GC)"

[2]	3GPP TS 33.501 "Security architecture and procedures for 5G System"

[3]	3GPP TS 23.501 "System architecture for the 5G System (5GS)"

[4]	3GPP TS 23.502 "Procedures for the 5G System (5GS)"

# 3	Rationale

The potential security requirement for Key Issue #10 "Authorization during Edge Data Network change" in TR 33.839 [1] is:

"Authorization of UE for EAS service access during Edge Data network relocation with seamless change shall be supported."

Currently, UE authorization for Data Network Access is performed by secondary authentication and authorization, as specified in TS 33.501 [2] clause 11.1 and TS 23.502 [4] clause 4.3.2.3. When a new PDU session is created for Edge Data Network relocation, secondary authentication and authorization will be triggered. If seamless change in the UE is required, session breakout can be used as it is not changing the IP address anchor of the PDU session, it only introduces an UL Classifier or multi-homing Branching Point (with added functionality to the PDU session). However, session breakout does not create a new PDU session, hence no new secondary authentication is required.

Hence the existing solutions specified in TS 33.501 [2] and TS 23.502 [4] already address the potential security requirement for Key Issue #10 "Authorization during Edge Data Network change" in TR 33.839 [1].

# 4	Detailed proposal

*** BEGIN CHANGES ***

### 7.10	Conclusions for Key Issue #10

Secondary authentication and authorization, as specified in TS 33.501 [7] clause 11.1 and TS 23.502 [5] clause 4.3.2.3, addresses authorization of UEs when the Edge Data Network is relocated. No new normative work is required.

*** END CHANGES ***