3GPP TSG-SA3 Meeting #104-e 	S3-212941

# e-meeting, 16 - 27 August 2021

# Source:	Ericsson

# Title:	Update to conclusion #1

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the pCR to TR 33.854 [1] below.

# 2	References

[1]	3GPP TR 33.854 "Study on security aspects of Unmanned Aerial Systems (UAS)"

[2]	3GPP TR 23.754 “Study on supporting Unmanned Aerial Systems (UAS) connectivity, Identification and tracking”

[3]	3GPP TS 23.256 “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”

# 3	Rationale

This contribution proposes to update conclusion #1 in TR 33.854 [1], to include the communication security between UAS-NF and USS/UTM. Usage of existing NEF/SCEF exposure services by UAS-NF is concluded in [2] and specified in [3]. Thus, the security mechanism should also be addressed in the conclusion in [1].

# 4	Detailed proposal

*** Start of Change ***

## 7.1	Conclusions for KI#1

UAV Authentication and Authorization (UAA) is recommended for the normative work based on the following solutions and principles:

NOTE: The agreement for normative work is on key common principles and not on the other details of the solutions.

- UAA is performed in 5G systems or EPS.

- UAA is performed between UAV and USS/UTM after Primary Authentication

- Revocation of UAV is initiated by USS/UTM using the 3GPP UAV ID

- UAA is performed either optionally during registration (5G solutions #1, #3, #7, #10 as basis) or during PDU session establishment (5G solution #5 as basis)

- USS/UTM is authorised to perform UAV authorization revocation, it is verified by UAS-NF (5G solutions #16 as basis)

- For EPS: solution #13 is chosen as the basis for normative work, with similar principles above

- In UAA, CAA Level UAV ID is used to identify UAV.

- Specific authentication methods for UAA are out of scope of 3GPP, the messages used for UAA exchanged between UAV and USS/UTM are included in transparent containers.

- Security related application layer information can be transported between UAV and USS/UTM in transparent containers (the content is out of scope of 3GPP).

- Existing security mechanism for NEF/SCEF exposure services is used for the secure communication between UAS-NF and USS/UTM.

*** End of Change ***