3GPP TSG-SA3 Meeting #104-e 	S3-212976

# e-meeting, 16 - 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.180 | CR | 0176 | rev | <Rev#> | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security for temporary calls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCXSec2 |  |  |  |  |  | Date: |  |  | 2021-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following procedures in TS 23.379 need to consider key distribution for the temporary group call and the regrouping operations: - Temporary group – broadcast group call procedure - Temporary group call – user regroup - Group regroup with preconfigured group - User regroup with preconfigured group For user regroup clause 7.3.2 of TS 33.180 specifies that the GMS supports creating new GMK and GMK-ID for regrouping users. For group regroup in clause 7.3.3 the GMS also supports creating new GMK and GMK-ID for the remporary regrouped group. To maximize the reuse of existing security framework and the alignment of existing architecture, the security enhancements with GMS based group keying have been taken into account in the proposed solutions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define new security procedures for the temporary call and preconfigured regroup procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No key distribution schemes for the temporary group call and the regrouping operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 7.3.a (new), 7.3.b (new), 7.3.c (new), 7.3.d (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CMS	Configuration Management Server

CS	Crypto Session

CSB-ID	Crypto Session Bundle Identifier

CSC	Common Services Core

CSK	Client-Server Key

CSK-ID	Client-Server Key Identifier

DPCK	MCData Payload Cipher Key

DPPK	MCData Payload Protection Key

DPPK-ID	MCData Payload Protection Key Identifier

GBA	Generic Bootstrapping Architecture

GCSK	Group Call Session Key

GCSK-RAND	Group Call Session Key RAND

GMK	Group Master Key

GMK-ID	Group Master Key Identifier

GMS	Group Management Server

GUK-ID	Group User Key Identifier

IdM	Identity Management

IdMS	Identity Management Server

InK	Integrity Key

InK-ID	Integrity Key Identifier

InterKMRec	Interworking Key Management Record

InterKMRec-ID	Interworking Key Management Record Identifier

InterSD	Interworking Security Data

IWF	InterWorking Function

JSON	JavaScript Object Notation

JWS	JSON Web Signature

JWT	JSON Web Token

KDF	Key Derivation Function

KFC	Key For Control Signalling

KFC-ID	Key for Floor Control Identifier

KMS	Key Management Server

MBCP	Media Burst Control Protocol

MCData	Mission Critical Data

MCPTT	Mission Critical Push to Talk

MCVideo	Mission Critical Video

MCX	Mission Critical Services

MKFC	Multicast Key for Floor Control

MSCCK	MBMS subchannel control key

MSRP	Message Session Relay Protocol

MuSiK	Multicast Signalling Key

MKI	Master Key Identifier

NTP	Network Time Protocol

NTP-UTC	Network Time Protocol – Coordinated Universal Time

OIDC	OpenID Connect

PCK	Private Call Key

PCK-ID	Private Call Key Identifier

PKCE	Proof Key for Code Exchange

PSK	Pre-Shared Key

SEG	Security Gateway

SeGy	Security Gateway

SPK	Signalling Protection Key

SRTCP	Secure Real-Time Transport Control Protocol

SRTP	Secure Real-Time Transport Protocol

SSRC	Synchronization Source

TBCP	Talk Burst Control Protocol

T-GID	Temporary Group Identifier

T-GMK	Temporary Group Master Key

T-GMK ID	Temporary Group Master Key Identifier

TGK	Traffic Generating Key

TrK	KMS Transport Key

TrK-ID	KMS Transport Key Identifier

UID	User Identifier for MIKEY-SAKKE (referred to as the 'Identifier' in RFC 6509 [11])

XPK	XML Protection Key

* * * Next Change * * * *

7.3.a	Temporary group call – user regroup security procedure

The security for temporary group call – user regroup as defined in TS 23.379 [2] is illustrate in Figure 7.3.a-1.

Figure 7.3.a-1: Temporary group call – user regroup security procedure

1-2.	These steps are as defined in sub clause 10.6.2.8.2 of 3GPP TS 23.379 [2].

3.	The MCPTT Server performs security procedure with the KMS based on procedures as defined in clause 5.3, by which the MCPTT Server is provisioned with the temporary group key T-GMK for end-to-end communication security for the temporary group call.

4.	The MCPTT Server requests user regroup operation by using group creation to the GMS acting as the group management client, carring the MC service user list requested in step 1 and the key material. The MCPTT server obtains a temporary group ID T-GID and associated group security information from the GMS.

5.	The MCPTT Server notifies the T-GID with associated group security information to the MCPTT client 1, 2 and 3.

6.	The MCPTT clients of the temporary user regrouped group send group regroup notification responses.

7.	The MCPTT Server affiliates users to group as defined in step 3 of sub clause 10.6.2.8.2 of 3GPP TS 23.379 [2].

8.	The MCPTT Server sends the group call requests towards the MCPTT clients. The message contains the temporary group indicator, T-GID and a randomly generated GCSK-RAND provided by the MCPTT Server. The group information (including T-GID, GCSK-RAND) is protected by the client-server key (CSK) provisioned by the KMS.

9-10.	These steps are as defined in sub clause 10.6.2.8.2 of 3GPP TS 23.379 [2].

11.	The MCPTT server sends the group call response to MCPTT client 1 to inform about successful call establishment. The message contains the GCSK-RAND.

12.	MCPTT client 1, 2 and 3 calculate the GCSK to be used for the upcoming group call session.

13.	MCPTT client 1, client 2 and client 3 establish media plane and floor control resources.

* * * Next Change * * * *

7.3.b	User regroup with preconfigured group security procedure

The security for user regroup with preconfigured group as defined in TS 23.379 [2] is illustrate in Figure 7.3.b-1.  .

Figure 7.3.b-1: User regroup with preconfigured group security procedure

1-3.	These steps are as defined in sub clause 10.6.2.12.2.1 of 3GPP TS 23.379 [2].

4.	The MCPTT Server performs security procedure with the KMS based on procedures as defined in clause 5.3, by which the MCPTT Server is provisioned with the temporary group key T-GMK for end-to-end communication security for the user regroup group.

5.	The MCPTT Server requests user regroup operation by using group creation to the GMS acting as the group management client, carring the MC service user list requested in step 1 and the key material. The MCPTT server obtains a temporary group ID T-GID and associated group security information from the GMS.

6.	The GMS may notify the group security information associated with the user regroup group to the MCPTT client 1, 2 and 3.

7.	If step 6 presents, the MCPTT clients of the user regroup group send group regroup notification responses.

8.	The MCPTT Server sends the preconfigured regroup requests towards the MCPTT clients. The message contains the user regroup group ID and a randomly generated GCSK-RAND provided by the MCPTT Server. The group information (including the group ID and GCSK-RAND) is protected by the client-server key (CSK) provisioned by the KMS.

9-10.	These steps are as defined in step 5 and 6 in sub clause 10.6.2.12.2.1 of 3GPP TS 23.379 [2].

11.	The MCPTT Server affiliates MCPTT clients to group as defined in step 7 of sub clause 10.6.2.12.2.1 of 3GPP TS 23.379 [2].

12.	The MCPTT server sends the preconfigured regroup response to MCPTT client 1 to inform about successful preconfigured user regroup. The message contains the GCSK-RAND.

13.	MCPTT client 1, 2 and 3 calculate the GCSK to be used for the subsequent group call session of the preconfigured user regroup group.

* * * Next Change * * * *

7.3.c	Temporary group call – broadcast group security procedure

The security for temporary group call – broadcast group as defined in TS 23.379 [2] is illustrate in Figure 7.3.c-1.

Figure 7.3.c-1: Temporary group call – broadcast group security procedure

1.	The MCPTT client of authorized user initiates a group call with multiple groups to the MCPTT server. A group call request message with the MCPTT group IDs (group1's MCPTT group ID, group2's MCPTT group ID) is routed to the MCPTT server.

2.	The MCPTT Server performs security procedure with the KMS based on procedures as defined in clause 5.3, by which the MCPTT Server is provisioned with the temporary group key T-GMK for end-to-end communication security for the temporary group call.

3.	The MCPTT Server requests group regroup to the GMS acting as the group management client, carring the MCPTT group IDs in step 1 and the key material. The MCPTT server obtains a temporary group ID T-GID and associated group security information from the GMS.

4.	The MCPTT Server notifies the T-GID with associated group security information to the MCPTT client 1, 2 and 3.

5.	The MCPTT clients of the temporary broadcast group send group configuration notification responses.

6.	The MCPTT Server sends the group call requests towards the MCPTT clients. The message contains the temporary group indicator, T-GID and a randomly generated GCSK-RAND provided by the MCPTT Server. The group information (including T-GID, GCSK-RAND) is protected by the client-server key (CSK) provisioned by the KMS.

7-8.	These steps are as defined in sub clause 10.6.2.5.3 of 3GPP TS 23.379 [2].

9.	The MCPTT server sends the group call response to MCPTT client 1 to inform about successful call establishment. The message contains the GCSK-RAND.

10.	MCPTT client 1, 2 and 3 calculate the GCSK to be used for the upcoming group call session.

11.	MCPTT client 1, client 2 and client 3 establish media plane.

12. The temporary group is torn down after the broadcast group call.