3GPP TSG-SA3 Meeting #105-e 	draft_S3-213871-r3

# e-meeting, 8 – 19 November 2021

Title:	Reply LS on Layer-3 UE-to-Network Relay authentication and authorization

Response to:	Reply LS (S3‑213871/S2-2107976) on Layer-3 UE-to-Network Relay authentication and authorization

Release:	Rel-17

Work Item:	FS_5G_ProSe_Sec

Source:	SA3

To:	SA2

Cc:	CT1

Contact person:	Soo Bum Lee

soobuml@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

SA3 would like to thank SA2 for the reply LS on Layer-3 UE-to-Network Relay authentication and authorization. SA3 discussed the reply LS and provides the following answers to SA2 questions:

Q1: Is the user plane solution (using a PKMF) to be documented only in SA3 specifications as in LTE ProSe design so that it does not require any change to the system architecture in TS 23.304? Does SA3 expect any support for the inter-PKMF interfaces, and would that need any SA2 documentation, or would it also be documented in SA3 specifications only?

- While no work is expected in SA2 for the user-plane solution, SA3 suggests SA2 to capture the new reference points introduced in 5G ProSe (i.e., between UE and PKMF and between PKMFs) in the SA2 specification.

Q2: SA2 has discussed control plane security solution but can't progress this part of the work without SA3 guidance. Can SA3 provide a consolidated control plane solution (based on primary authentication), or a list of expected system impacts for SA2 to incorporate the corresponding enhancements to TS 23.304?

- The followings are the list of the agreed (high-level) procedures.

  - Remote UE’s primary authentication via the Relay UE’s NAS connection. In other words, the Relay UE’s AMF should support the primary authentication of the Remote UE over the existing NAS connection with the Relay UE.

  - PC5 key derivation based on the Remote UE’s KAUSF. This requires the Relay UE’s AMF interacts with the Remote UE’s AUSF to fetch the PC5 key.

- SA3 would like to inform that the latest control plane solution based on the agreed procedures is documented in TS 33.503. SA3 will keep SA2 informed on the further progress.

# 2	Actions

To SA2

ACTION: 	3GPP TSG SA WG3 would like SA2 to take the above feedback into account.

# 3	Dates of next TSG SA WG3 meetings

SA3#106	            07 – 11 February 2022		                e-meeting

SA3#106-bis	            04 – 08 April 2022			   TBD