3GPP TSG-SA3 Meeting #105-e	S3-213928

# e-meeting, 8 - 19 November 2021

# Source:	CableLabs

# Title:	Update Nnssaaf_AIW interface

# Document for:	Approval

Agenda Item:	4.17

# 1	Decision/action requested

It is proposed to approve the changes to be included in the draft-CR for eNPN

# 2	References

# 3	Rationale

This contribution updates Nnssaaf_AIW to support AVP container so that the interface is extensible.

# 4	Detailed proposal

*****FIRST CHANGE*****

### 14.4.X	Nnssaaf_AIW services

#### 14.4.X.1	General

The following table illustrates the security related services provided by the NSSAAF for primary authentication in SNPN with Credentials holder using AAA server (see section I.2.2.z).

Table 14.4.X.1-1: NF services for CH using AAA for primary authentication provided by NSSAAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnssaaf_AIW | Authenticate | Request/Response | AUSF |

#### 14.4.X.2	Nnssaaf_AIW_Authenticate service operation

Service operation name: Nnssaaf_AIW_Authenticate

Description: The NSSAAF provides Authentication and Authorization service to the consumer NF by relaying EAP or EAP-TTLS inner method messages towards a AAA Server and performing related protocol conversion as needed.

Input, Required:

1) In EAP Authentication:

a) In the initial authentication request: SUPI.

b) In subsequent authentication requests: EAP message.

2) In case EAP-TTLS mechanisms are implemented: inner method container.

Input, Optional: None

Output, Required:

1) In EAP authentication: EAP message, authentication result and if success MSK.

2) In case EAP-TTLS mechanisms are implemented: inner method container.

Output, Optional: None

**** END OF CHANGES*****