3GPP TSG-SA3 Meeting #105-e 	draft_S3-213970-r1

# e-meeting, 8 - 19 November 2021

# Source:	LG Electronics, Interdigital

# Title:	Conclusion for secondary authentication

# Document for:	Approval

Agenda Item:	5.7

1	Decision/action requested

This contribution proposes conclusion text for secondary authentication in TR 33.847

2	References

[1]	TR 33.847 v0.8.0

3	Rationale

This document presents conclusions for the support for secondary authentication of Remote UE via L3 U2N relay. There are three potential solutions for the support of secondary authentication in [1]: Solution #13, Solution #25 and Solution #34.

Sol#13:

- The existing secondary authentication mechanism cannot be reused as it is

  - SMF in the Relay UE's network has to trigger the Remote UE secondary A&A based on the Remote UE's subscription and the local policy

  - The existing EAP authentication messages for the secondary authentication are expected to be reused as many as possible

- The Remote UE and the Relay UE have to support EAP authentication procedure over PC5 interface

- This solution can work with both CP based and UP based Remote UE authorization. However how the SMF obtains Remote UE's SUPI when UP based solution is considered is unclear

Sol#25:

- The existing secondary authentication mechanism cannot be reused as it is

  - SMF in the Relay UE's network has to trigger the Remote UE secondary A&A based on the Remote UE's subscription

  - The existing EAP authentication messages for the secondary authentication are expected to be reused as many as possible

- The Remote UE and the Relay UE have to support EAP authentication procedure over PC5 interface

- CP based Remote UE authorization is mandated

Sol#34:

- The existing secondary authentication mechanism can be reused as it is

- N3IWF is always required to support the secondary A&A even when the E2E security is not needed

- If VPLMN(Relay UE's network) doesn't implement the N3IWF, the remote UE has to connect to HPLMN's N3IWF to perform the secondary A&A, which may cause traffic delay.

Proposal:

- It is proposed to support the secondary authentication of the Remote UE via L3 UE-to-Network Relay as it may be required by the DN

- It is proposed to select solution #13/#25 and solution #34 for the support of secondary authentication, respectively without and with N3IWF support.

4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.847 [1].

***	BEGIN OF CHANGES	***

7.4	Key issue #4: Authorization in the UE-to-Network relay scenario

The solutions for U2N Relay authorization and security can be classified as user-plane (UP) or controlled-plane (CP) based solutions. The UP based solutions use a UP connection to a PKMF while CP based solutions use the primary authentication for PC5 keys establishment.

The following text is taken as conclusions for the UE-to-Network Relay solution:

-	For the control plane solution:

-	Baseline solution for Authorization for Remote UE/Relay is based on primary authentication (CP based approach, e.g., sol#1, #10, #15, #30) and using PCF based service authorization and provisioning as defined in [16] TS 23.304 clause 5.1.4.

-	Performing primary authentication during PC5 link establishment is supported (e.g. Sol#1, Sol#10, Sol#30).

NOTE:	The detailed procedure to enable authorization for Remote UE/Relay will be determined accordingly during normative phase. Additional support for Remote UE using its 5G-GUTI in DCR will be determined during normative phase.

-	For the user-plane solution, based on the conclusions in KI #3, it is concluded that the user-plane solutions including Solution #18, Solution #21 and Solution #29 are selected as the basis of normative work.

-	For the support of secondary authentication it is concluded that both solutions without and with N3IWF are supported for L3 U2N relay. Solution #13 / Solution #25 (without N3IWF) and Solution #34 (with N3IWF) are selected as the basis of normative work.

NOTE:	The detailed procedure to enable Secondary Authentication for Remote UE without N3IWF will be determined during normative phase with coordination with SA2.

Editor's note:	Final conclusion for NSSAA is FFS.

***	END OF CHANGES	***