3GPP TSG-SA3 Meeting #105-e 	draft_S3-214422-r2

# e-meeting, 08 – 19 November 2021                                              Revision of S3-214009

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draft-CR | rev | - | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Living CR for 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It’s the living CR with the Security aspects for the MBS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The contribution adds skeleton and security aspects related to the MBSF/MBSTF and the interface. Security mechanisms for MBS traffic transmission are added as well. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security aspects are absent for MBS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex clause |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************ START OF 1st CHANGE*******

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[xx]	3GPP TS 33.246: “Security of Multimedia Broadcast/Multicast Service (MBMS)".

[yy]	3GPP TS 23.247: “Architectural enhancements for 5G multicast-broadcast services".

[zz]	3GPP TS 33.535: “Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

************ END OF 1st CHANGE*********

************ START OF 2nd CHANGE******

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RG	5G Residential Gateway

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

CCA	Client Credentials Assertion

Cell-ID	Cell Identity as used in TS 38.331 [22]

CHO	Conditional Handover

CIoT	Cellular Internet of Things

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

CP	Control Plane

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EDT	Early Data Transmission

EMSK	Extended Master Session Key

ENSI	External Network Slice Inforamtion

EPS	Evolved Packet System

FN-RG	Fixed Network RG

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPUPS	Inter-PLMN UP Security

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LI	Lawful Intercept

MBSF	Multicast/Broadcast Service Function

MBSTF	Multicast/Broadcast Service Transport Function

MN	Master Node

MO-EDT	Mobile Originated Early Data Transmission

MT-EDT	Mobile Terminated Early Data Transmission

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G-Capable

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

NSSAA	Network Slice Specific Authentication and Authorization

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

PUR	Preconfigured Uplink Resource

QoS	Quality of Service

RES	RESponse

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SCP	Service Communication Proxy

NOTE: Void.	Security Gateway

SEPP	Security Edge Protection Proxy

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

TSC	Time Sensitive Communication

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

USIM	Universal Subscriber Identity Module

XRES	eXpected RESponse

************ END OF 2nd CHANGE********

************ START OF 3rd CHANGE******

Annex X (normative): Security for Multicast/Broadcast Service for 3GPP service

# X.1		General

Editor's Note: This clause will describe the general part on Multicast/Broadcast Service.

This clause describes the security requirements, procedures and handling for 5G Multicast/Broadcast Service (MBS).The general features for 5G MBS are described in TS 23.247 [yy].

NOTE: Security for Multicast-broadcast service for roaming is not supported in this release.

## X.2		Security requirements

Editor's Note: This clause will describe security requirements and features of Multicast/Broadcast Service.

## X.2.1	Requirements of MBSF

The security requirements on the NEF described in clause 5.9.2.3 of present specification also apply to MBSF.

## X.2.2	Requirements of MBSTF

The security requirements on the NEF described in clause 5.9.2.3 of present specification also apply to MBSTF.

X.3		Security mechanisms for xMB-C/MB2-C and xMB-U/MB2-U interface

The security aspects defined in clause 12 in present specification is applicable for both MBSF and MBSTF. TLS based solution are reused to protect the interface xMB-C/MB2-C and xMB-U/MB2-U between AF and 5GC in MBS.

X.4		Security mechanisms for MBS traffic transmission

X.4.1		Key derivation, management and distribution

For security protection of MBS traffic, control-plane procedure and user-plane procedure are optionally supported in service layer. The multicast security policy between UE and RAN shall be not needed to avoid redundant protection.

For security protection of MBS traffic, control-plane procedure and user-plane procedure are optionally supported in service layer. The multicast security policy between UE and RAN shall be not needed to avoid redundant protection.

X.4.1.1	Control-plane procedure

The multicast session security context consists of the MBS session ID, MBS keys and the corresponding key ID. The MBS keys include MBS Service Key (MSK) and MBS Traffic Key (MTK). MBS traffic is protected with the MTK. The MSK is used to protect the MTK when the MTK is delivered to the UE. The MSK ID and MTK ID are determined as specified in Clause 6.3.2.1 and clause 6.3.3.1 of TS 33.246 [xx].

The MBSF generates the MSK and its key ID for a MBS session and distributes the MSK to the MB-SMF and MBSTF. The MBSF shall distribute them to MB-SMF either upon request by the MB-SMF (i.e., pull) or when a new MSK is generated (i.e., push). The MBSF may also include the MSK lifetime when it distributes the MSK to MBSTF.

The MBSTF generates the MTK and its key ID for the MBS traffic protection. A new MTK may be generated based on the MBS session security policy. When the MBSTF generates a new MTK, the MBSTF shall multicast the MTK after protecting it using the MSK as specified in TS 33.246 [xx]. The MBSTF shall also provide the new MTK to the MBSF.

In the multicast session join and session establishment procedure, the SMF interacts with the MB-SMF to retrieve the multicast session security context. The SMF shall provide the multicast session security context to the UE if the UE is authorized to use the required multicast service. The UE uses the received MTK to process the protected MBS traffic until it receives a new MTK update over the user-plane.

The MSK may be updated based on the request from MB-SMF or AS (e.g., due to the change of authorization information) or based on the local policy (e.g., key lifetime expiration). When the MSK is updated, the MBSF shall send the new MSK to the MB-SMF and then the MB-SMF shall trigger the session update as specified in clause 7.2.6 in TS 23.247 [yy]. The MSK and the corresponding key ID are delivered to the UEs that has joined the multicast session.  The MBSF shall also send the new MSK to the MBSTF. The MBSTF may request a MSK to the MBSF when it does not have a valid MSK (e.g., due to the current MSK expiration).

The MTK may be updated based on the change of the authorization information or based on the local policy (e.g. key lifetime expiration). In such cases, the MBSF or MB-SMF may trigger the MTK update to the MBSFT. The key update request message shall include the MBS session ID. If the MBSFT has generated a new MTK, the MBSFT shall provide the new MTK to the MBSF. To improve the efficiency of MTK update, the updated MTK is delivered from MBSTF to the UE using MIKEY over UDP as specified in clause 6.3.3.2 in TS 33.246 [xx]. The MSK is used to protect the updated MTK. The UE shall not send an error message to the MBSTF as a result of receiving an MTK message.

X.4.1.2	User-plane procedure

The UE registers to the MBS service and receives the MBS traffic as specified in TS 33.246 [xx] with the following changes.

- MBSTF takes the role of the BM-SC in TS 33.246 [xx].

- The UE authenticates to the MBSTF based on the GBA as in MBMS security (see TS 33.246 [xx]) or based on the AKMA (see TS 33.535 [zz]). When the AKMA is used, the MRK is derived from the KAF as specified in Annex F of TS 33.246 [xx] by replacing the Ks_NAF for the GBA_ME run with KAF. Furthermore, when the AKMA is used, the MUK is set to KAF.

Editor’s Note: When the AKMA is used, how the MBSTF obtains the authorization information is FFS.

X.4.2		Protection of the traffic transmission

The actual method of protection may vary depending on the type of data being transmitted, e.g. media streaming application or file download. Clause 6.6.2 and clause 6.6.3 in TS 33.246 [xx] apply to the protection of streaming data and protection of download data, respectively.

X.4.3		Authentication and authorization aspects for the multicast session

The support for the optional-to-use authentication and authorization procedure for a 5G multicast session is specified in this clause.

NOTE: Roaming is not supported for 5G MBS according to TS 23.247 [yy].

Editor's Note: secondary authentication procedure for multicast PDU session will be added if confirmed by SA WG2.

AKMA/GBA is supported for authentication and authorization in user-plane procedure for security protection of MBS traffic, as specified in clause X.4.1.2 of present document.

X.5  Security protection for interworking between 5MBS and eMBMS

Interworking between 5G MBS and eMBMS is supported at service layer.  The procedures for inter system mobility with interworking at service layer is specified in clause 7.4 in TS 23.247 [yy].

The joint BM-SC+MBSF/MBSTF functionality provides the security protection for MBS traffic. During inter-system mobility, when the target system is EPS, the security protection specified in TS 33.246 [xx] applies. The security protection specified in present document applies to the case when the target system is 5GS.

************ END OF 3rd CHANGE*********