3GPP TSG-SA3 Meeting #106-e	S3-220116r1

# e-meeting, 14 – 25 February 2022											Merger of S3-220116 and 220200

# Source:	Huawei, HiSilicon, Xiaomi

# Title:	update to KI#2

# Document for:	Approval

Agenda Item:	5.12 FS_ eNS2_SEC

# 1	Decision/action requested

Approve the proposed updates to KI#2 for TR33.874

# 2	References

[1]	S2-2107942 Reply LS on NSAC procedure

# 3	Rationale

In the LS [1], SA2 has provided answers to three questions that SA3 raised before. This contribution proposes to update the KI#2 based on the answers and observations, as summarized below:

- [SA2] SA2 would like to indicate that the UE usage of a user services is not considered when counting the number of UEs registered to a network slice

Observation 1: UE usage has not been considered which will cause the problem pointed out in this key issue. SA3 should address this problem and provide potential solutions.

- [SA2] The NSSAA status is kept in the UE context in AMF (also transferred between AMFs), i.e. the AMF may avoid NSSAA when NSSAA has been successful for an S-NSSAI.

Observation 2: The previous NSSAA status is kept (e.g. successful) and extra NSSAA can be aoived. There should be no problem with the clarification.

- [SA2] If needed, the operator may activate EAC mode when the current count is reaching certain operator defined threshold.

Observation 3: The problem of “burst in the number of UEs” has been validated. It is suggested that the problem could be mitigated at time of setting the EAC-mode threshold, taking into consideration the burst problem.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES   ***

## 5.2	Key Issue 2: DoS to NSAC procedure and circumvention of admission control

### 5.2.1	Key issue details

A new Network Slice Admission Control (NSAC) procedure has been introduced in TS23.501 [2] and TS23.502 [3], where the number of registered UEs is monitored for a network slice (i.e. S-NSSAI) and a UE will be rejected to access if the number of UE registered in the requested S-NSSAI has reached its quota.  However, the NSAC procedure needs to be studied further to address potential security risks, for examples:

- In the current NSAC procedure, the number of registered UE in an S-NSSAI is updated independently from other S-NSSAIs during the registration procedure. In other words, the granularity level at registration is S-NSSAI. However, it is not the case in the de-registration procedure. The numbers are only updated when the UE exits from all network slices, i.e. de-registered. Since a UE may access multiple slices, e.g. eight, the UE would still be counted against quota usage of ALL S-NSSAIs even the UE is not using some or most of slices (“idly occupied” by the UE). This may lead to the quota reached fast which does not reflect the real usage of a slice. Other legitimate UEs will suffer from DoS – “dog in the mager”. It is notable that an attacker can use legitimate UEs to launch such attacks.

- The Early Admission Control (EAC) mode has been introduced where the admission control can be inactive if the number of UE bellows a pre-configured threshold. This may pose a security risk that exceeds the slice quota when a sudden increase in the slice registration requests, maliciously or accidentally.

### 5.2.2	Security threats

If the NSAC procedure does not reflect the real situation of the slice usage, an attacker may launch a DoS attack to legitimate users.  The DoS may also happen inadvertently when many UEs do not use the slices registered.

If EAC mode is not activated properly, it will cause circumvention of admission control DoS to the network slices.

### 5.2.3	Potential security requirements

The 5G system should provide mechanisms to prevent DoS due to inconsistency between “slice registration” and “slice usage” by UEs.

The 5G system should prevent circumvention of admission control DoS due to the EAC inactive mode.

***	END OF CHANGES   ***