# 3GPP TSG SA WG3 Meeting 106-e	draft_S3-220152-r3

# Electronic meeting, 14-25 February 2022

Title:	Reply LS on Security of Small data transmission

Response to:	LS S3-210051(R2-2201983) LS on Security for Small Data Transmission

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	SA3

To:	RAN2

Cc:

Contact person:	Abhijeet Kolekar

abhijeet.kolekar@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

SA3 thanks RAN2 for their LS on Small data transmission.

SA3 discussed the LS from RAN2 and would like to provide the following response on the exemplary call flow detailed in the RAN2 LS (S3-220051/R2-2201983):

- Two nodes (gNBs) using a key (KRRCint_1) to verify RRC messages (at step 12 & 13 in serving gNB and at step 18 in last serving gNB) is not acceptable from security point of view. RAN2 need to alignthe solution with the legacy procedure to prohibit the use of same key at different entities.

- If there is a scenario for multiple RRC messages using the same I-RNTI (as in the case of step 16 for non-SDT), the procedure should make sure a fresh I-RNTI is assigned to the UE similar to the legacy procedure.

- The gNB shall perform a vertical key derivation in case it has an unused {NH, NCC}, in order to follow the forward security principle (c.f., S3-182541). RAN2 need to update the solution to use vertical key derivation in case gNB has an unused {NH, NCC}.

- There is SA3 specification work needed to specify the security solution, if RAN2 agrees for the solution similar to the one mentioned in the RAN2 LS (S3-220051/R2-2201983).

The exemplary call flow detailed in the RAN2 LS (S3-220051/R2-2201983) is feasible from security point of view, if the above mentioned security concerns a), b) and c) are addressed by RAN2.

# 2	Actions

To RAN2

ACTION: 	3GPP TSG SA WG3 asks RAN2 to take the above feedback into account and reply to SA3.

# 3	Dates of next TSG SA WG3 meetings

TSG SA WG3 Meeting 106-Bis-e	04-08 April 2022

TSG SA WG3 Meeting 107-e	27 June-01 July 2022