3GPP TSG-SA3 Meeting #106e 	S3-220197-r1

e-meeting, 14 - 25 February 2021

Title:	REPLY LS on support of DCS variants in UE Onboarding Architecture.

Release:	Release 17

Work Item:	eNPN

Source:	SA3

To:	SA2

Cc:

Contact person:	Bo Bjerrum

bo dot bjerrum at nokia dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 would like to thank SA2 for supporting the conclusions drawn by agreeing to the two variants of DCS’s.

SA3 confirms that DCS deployments can support both AAA or AUSF/UDM functionality. AAA connected over e.g. radius or Diameter and AMF to AUSF over SBI.

SA3 agrees that  discovery and selection of either AUSF or AAA based primary authentication can be handled as already specified in TS 23.501 and TS 23.502.

It shall be noted that the configuration of the credentials implicitly decides whether to use AAA or AUSF/UDM based DCS and ON-SNPN selection configuration shall align with that.  It’s not possible to select both AAA and AUSF/UDM based DCS with the same credential configuration.

Unrelated to above, SA3 proposes to have the UDM selection optional and leave this to the choice of the ON-SNPN deployment. The reasoning for the optionality is, that in some usages the network needs to be self-contained and access to external resources is limited, like the DCS. One example is networks handling operational technology in industrial setups standardised by the IEC 62443 series. In these usages the default credentials are recommend being stored in UDM to avoid fragmentation from processes already define in TS 33.501.

# 2	Actions

To TSG SA WG2

ACTION: 	SA3 kindly asks SA2 to take above information into account and provide any feedback, if needed.

# 3	Dates of next TSG SA WG 3 meetings

SA3#106-bis	4 – 8 April 2022		Online

SA3#107-bis	27 June – 1 July 2022	online

SA3#108	22 – 26 August 2022	Goteborg, SE