3GPP TSG-SA3 Meeting #106-e 	S3-220329

# e-meeting, 14 - 25 February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.847 | CR | 0004 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional conclusion of KI #17 – security policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, CATT, InterDigital, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_ProSe_Sec |  |  |  |  |  | Date: |  |  | 2022-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Conclusion not provided for key #17 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of conclusion for key issue #17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing conclusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** START OF CHANGES *****

7.17	Key Issue #17: Supporting security policy handling for PC5 connection of 5G ProSe services

The following text is taken as conclusions for the security policy handling:

- Security policy provisioning:

  - For user-plane UE-to-network relay solutions, the PKMF provides PC5 security policies to the UE.

NOTE: PKMF may get the security policies in different ways (e.g., from PCF, from ProSe Application server, or based on local configuration) and this will be decided in the normative phase.

  - For control-plane UE-to-network relay solutions, the PCF provides PC5 security policies to the UE as per TS 33.536 [8].

  - For ProSe direct services, the PCF provides PC5 security policies to the UE as per TS 33.536 [8].

- The security policy negotiation and enforcement for PC5 connection is based on the TS 33.536 [8].

- The security policy for signaling integrity protection on PC5 is set to “REQUIRED” for all UE-to-network relay scenarios.

- The security policy for user plane protection on PC5 for ProSe UE-to-network relay scenarios depends on the specific relay service requested by the UE.

***** END OF CHANGES *****