3GPP TSG-SA3 Meeting #107-e 	S3-220689

# e-meeting, 16 - 20 May 2022

# Source:	Keysight Technologies

# Title:	New test case for confidentiality, integrity and replay protection between AAnF and AUSF

# Document for:	Approval

Agenda Item:	4.5

# 1	Decision/action requested

We ask SA3 to accept a new test specification about traffic protection in TS 33.537.

# 2	References

# 3	Rationale

Clause 4.4.0 in TS 33.535 includes this security requirement:

- The SBA interface between the AAnF and the AUSF shall be confidentiality, integrity and replay protected.

It is proposed to include a new test case to cover this security requirement of the confidentiality, integrity and replay protection of the communication between AAnF and AUSF.

As TS 33.501 – clause 13.1 defines the communications between NFs in the SBI must be protected by TLS/mTLS, the test case makes reference to that procedure in TS 33.117 – 4.2.2.2.2.

# 4	Detailed proposal

***** BEGIN OF CHANGE *****

##### 4.2.3.2.4	Protecting data and information in transfer

##### 4.2.3.2.4.1	Confidentiality, integrity and replay protections over SBA interface

Requirement Name: Confidentiality, integrity and replay protections over SBA interface

Requirement Reference: TS 33.535 [X], clause 4.4.0

Requirement Description: "The SBA interface between the AAnF and the AUSF shall be confidentiality, integrity and replay protected." as specified in TS 33.535 [X], clause 4.4.0

Threat References: TR 33.926 [X], clause X.1.1, Control plane data protection with AUSF

Test Case:

Test Name: TC_PROTECT_SBA_AAnF_AUSF

Purpose:

Verify that the transported data between AAnF and AUSF are confidentiality, integrity and replay protected over SBA interface.

Pre-Conditions:

- AAnF and AUSF network products are connected in simulated/real network environment.

- Network product documentation containing information about supported TLS protocol and certificates is provided by the vendor.

- Tester shall have access to the SBA interface between AAnF and AUSF.

Execution Steps:

The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.2.2.2 of TS 33.117 [X].

Expected Results:

The user data transported between AAnF and AUSF is confidentiality, integrity and replay protected.

Expected format of evidence:

Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture or pcap traces.

***** END OF THE CHANGE ****