3GPP TSG-SA3 Meeting #108Adhoc-e	draft_S3-222478-r2

# e-meeting, 10th - 14th October 2022

# Source:	Huawei, HiSilicon

# Title:	New solution of security for the Ranging/SL positioning device discovery

# Document for:	Approval

Agenda Item:	5.19

1	Decision/action requested

Approve the new solution proposal to TR 33.893

2	References

N/A

3	Rationale

The contribution proposes to add a new solution to provide secure Ranging/SL positioning service discovery.

4	Detailed proposal

***BEGINNING OF THE 1st CHANGE***

6.X	Solution #X: Security of the Ranging/SL positioning device discovery

6.X.1	Introduction

This solution addresses the security of the discovery procedures for Ranging/SL Positioning service as specified in Key Issue #3. As per the assumption under clause 4.1, the solution of discovery security reuses the direct discovery security defined for 5G ProSe in TS 33.503 [6] as a baseline and adjusts to the Ranging/SL Positioning scenario.

6.X.2	Solution details

The discovery procedure for the Ranging/SL positioning service is classified as restricted discovery, considering the discovery procedures happen between UEs with dedicated Ranging/SL positioning service authorisation (i.e. discover with explicit permission).

The information exchanged during PC5 direct discovery for Ranging/SL Positioning service may include security/privacy sensitive information (e.g. ranging role informtion of Ranging/SL Positioning UE). Thus integrity and confidentiality protections are required by the Ranging/SL positioning discovery. Based on the clause 6.1.3.2.2 in TS 33.503 [6], the solution proposes the following modifications for the Ranging/SL positioning discovery:

- the Code-Sending Security Parameters received from 5G DDNMF always include a DUCK and corresponding Encrypted_bits_mask for confidentiality protection, and a DUIK for integrity protection.

- the Code-Receiving Security Parameters received from 5G DDNMF always include a DUCK and corresponding Encrypted_bits_mask for confidentiality protection, the the Code-Receiving Security Parameters include the DUIK.

Editor’s NOTE: whether or not to reuse 5G DDNMF in 5G ProSe to support the Ranging/SL positioning service discovery will be decided by SA2.

6.X.3	Evaluation

TBD.

***END OF THE CHANGES***